BAHIRDAR UNIVERSITY

COLLEGE OF AGRICULTURE AND ENVIRONMENT SCIENCE

GRADUATE PROGRAM

MODELLING THE EFFECT OF LAND USE LAND COVER CHANGES ON SOIL EROSION AND SEDIMENT YIELD IN MAYBAR WATERSHED NORTHEAST ETHIOPIA

M.Sc. THESIS

BY

YEMIYAMIR MARIYE ARAGAW

June 2023

Bahirdar, Ethiopia

BAHIRDAR UNIVERSITY

COLLEGE OF AGRICULTURE AND ENVIRONMENT SCIENCE

GRADUATE PROGRAM

MODELLING THE EFFECT OF LAND USE LAND COVER CHANGES ON SOIL EROSION AND SEDIMENT YIELD IN MAYBAR WATERSHED NORTHEAST ETHIOPIA

M.Sc. THESIS

BY

YEMIYAMIR MARIYE ARAGAW

A THESIS SUMMITTED TO THE DEPARTMENT OF NATURAL RESOURCE MANAGEMET IN PARTIAL FULFILLMENT OF THE REQUIREMENTS FOR THE DEGREE OF MASTER OF SCIENCE IN ENVIRONMENT AND CLIMATE CHANGE

June 2023

Bahirdar, Ethiopia

THESIS APPROVAL SHEET

As member of board of examiner of Master of Science (M.Sc) thesis open defense examination, we have read and evaluated this thesis prepared by MR. YEMIYAMIR MARIYE ARAGAW entitled MODELLING THE EFFECT OF LAND USE LAND COVER CHANGES ON SOIL EROSION AND SEDIMENT YIELD IN MAYBAR WATERSHED NORTHEAST ETHIOPIA we here by certify that, the thesis is accepted for fulfilling the requirement of the award of master of science (M.Sc) in environment and climate change.

Board of Examiners:

1. D/r HAILU KENDIE                         _______________         _______________

Name of External examiner                     Signature	                      Date

2. D/r MULATE MEKONNEN	    _______________           _______________

Name of internal examiner                   Signature	                         Date

3. D/r ANWAR ASSEFA                               ________________

Name of Chairman                             Signature          		   Date

DECLARATION

This is to certify that thesis entitled “MODELLING THE EFFECT OF LAND USE  LAND COVER CHANGES ON SOIL EROSION AND SEDIMENT YIELD IN MAYBAR WATERSHED NORTHEAST ETHIOPIA” submitted in partial fulfillment of the requirements for the award of the degree of master of sciences in “Environment and climate change” to the graduate program of college of agriculture and environmental sciences, Bahire Dar University by Mr. YEMIYAMIR MARIYE ARAGAW (ID No BDU 1207752PR) is an authentic work carried out by him under our guidance. The matter embodied in this projects work has not been submitted earlier for award of any degree or diploma to the best of our knowledge and belief

Name of the Student

Yemiyamir Mariye                  __________________         ________________

Signature		                 Date

Name of Major Advisor

Lewoye Tsegaye (Associate.prof)    _____________	________________

Signature	                   Date

DEDICATION

This M.Sc Thesis is dedicated to my mother, the late ALMAZE BIYABEYEN ASFAW, for bringing me in education.

# ACKNOWLEDGEMENTS

My first gratitude is to the Almighty God, who gave me health and strength during the course of this study and in my entire life. I would like to extend my deepest gratitude to my major advisor, LEWOYE TSEGAYE (Associate.prof), for his support and guidance starting from problem formulation to the end of this research. I am also delighted to express my out most gratitude to TILAHUN TAYE for supportin my research work. I greatly acknowledge Combolcha metrological and Sirenka research center for providing rainfall, sediment and streamflow data the agricultural development agents and local elders of the watershed for their interest to providing necessary information.

I am also happy to acknowledge Gubalafto wored agricultural office for giving the time to extensively follow the program, allowing me their offices to use internet and supporting my thesis work by printing. I extend my deepest gratitude to my father, my sisters and my brothers for their encouragement and moral support in the course of my life. Last but not least, I acknowledge my wife TIRUWORK DEMEKE SISAY and my Son SARA YEMIYAMIR MARIYE for providing additional efforts for this work.

# LIST OF ACRONYMS

ArcGIS                                       Aeronautical reconnaissance coverage Geographic

Information System

ASTERDEM                               Aster Digital Elevation Model

CN                                               Curve Number

ETM+                                                                Enhanced Thematic Mapper plus

GIS                                              Geographical Information System

| GPS | Global position system |
| --- | --- |

HRUs                                           Hydrologic Response Units

IDW                                              Inverse Distance Weighting

InVEST                                         Integrated Valuation of Ecosystem Services and

Tradeoffs

LULC                                             Land Use and Land Cover

masl                                                Meter Above Sea Level

MoME                                             Ministry of Mines and Energy

M.Sc                                                Master of Science

NDVI                                             Normalized Difference Vegetation Index

NMA                                               National Meteorological Agency

NWIS                                             National Water Information System

OLI                                                  Operational Land Imager

OA                                                   Overall Accuracy

PA                                                     Producer’s Accuracy

RUSLE                                             Revised Universal Soil Loss Equation

SCS                                                   Soil Conservation Service

SDR                                                  Sediment Delivery Ratio

SCRP                                                Soil Conservation Research Project

SRC                                                  Sirinka Research Center

TIRS                                                 Thermal Infrared Sensor

TM                                                   Thematic Mapper

UA                                                    User’s Accuracy

USGS                                                    United States Geological Survey

UTM                                                 Universal Transverse Mercator

WGS                                                 World Geodetic System

MODELLING THE EFFECT OF LAND USE LAND COVER CHANGES ON SOIL EROSION AND SEDIMENT YIELD IN MAYBAR WATERSHED NORTHEAST ETHIOPIA

BY: - YEMIYAMIR MARIYE ARAGAW Advisor: - LEWOYE TSEGAYE (Ass. prof)

ABSTRACT

The effects of (LULC) change on soil erosion and sediment yield is getting global attention. Soil erosion is one of the critical environmental problems worldwide with high severity in developing countries. Understanding the effects of LULC changes on soil erosion and sediment yield is not only to identify challenges and targeting interventions but also for watershed management planning. This study investigated the effects of LULC changes on soil erosion and sediment yield in the maybar watershed during the 1990- 2020 periods. The hybrid land use classification technique for classifying time series Land sat images (1990, 2000, 2010 and 2020) were used. The land use types in the study area were categorized into five cultivated, dense forest, grazing, open forest and settlement. The InVEST SDR model for estimating the rate of soil erosion and sediment yield were employed in the study. The results showed that in the past three decades, dense forest and settlement areas increased with 23ha, 29.31ha, 42.65ha, 45.44ha and 15ha, 21.36ha, 22.4ha, 24ha in the study period of 1990, 2000, 2010 and 2020 respectively. While areas occupied by grazing land and open forest land decreases from 12ha, 6.4ha, 3ha, 2ha and 36.6ha, 32ha, 24ha, 19.2ha in the study period of 1990, 2000, 2010 and 2020 respectively. On the other hand cultivated land decreased 29.67ha, 27.2ha, 24.22ha in the study period of 1990, 2000 and 2010 respectively but increased 25.63ha in 2020 study period. The LULC changes have also decrease the annual mean soil erosion rate from 28.9t/ ha/year in 1990 25.12 t/ha /year in 2000 and 21.06 t/ha/year in 2010 and increase in the year of 2020 to 26.16 t/ha/year. The sediment yield is also decrease from 6.4t/ ha/year in 1990, 4.8 t/ha/year in 2000 and 3.6 t/ha/year in 2010 and increase in the year of 2020 to 5.07 t/ha/year. Information on soil loss and sediment export is essential to conservation interventions in a given watershed.

Keywords: - Land Use Land Cover change, Accuracy Assessment, InVEST SDR model, Maybar, Validation.

TABLE OF CONTENTS

Table of Content

ACKNOWLEDGEMENTS	vi

LIST OF ACRONYMS	viii

LIST OF TABLES	xiii

LIST OF FIGURES	xiv

1. INTRODUCTION	1

1.1. Background	1

1.2. Statement of the Problem	2

1.3. Objectives of Study	5

1.3.1. General objective	5

1.3.2. Specific Objective	5

1.4. Research Questions	5

1.5. Scope of the Study	5

1.6. Limitation of the Study	5

1.7. Significance of the Study	5

2. LITERATURE REVIEW	7

2.1. Soil Erosion	7

2.1.1. Global Context of Soil Erosion	8

2.1.2. Soil Erosion in Ethiopia	9

2.2. Land Use Land Cover Change	10

2.2.1. Land Use Land Cover Change at Global Perspective	10

2.2.2. Land Use and Land Cover Change in Ethiopia	11

2.3. Sediment Yield	14

2.4. Description of Soil Erosion Models	15

2.4.1. Invest SDR Model	15

2.4.2. Revised Universal Soil Loss Equation (RUSLE) Model	15

3. MATERIALS AND METHODS	18

3.1. Study Area Description	18

3.2. Data Types, Sources and Analysis	19

3.2.1. Satellite data source and processing	19

3.3. Image Classification Methods	20

3.4. Accuracy Assessment and Change Detection	21

3.5. Invest SDR Model Data Inputs	23

3.5.1. Watershed Boundary	23

3.5.2. LULC Data	23

3.5.3. Rainfall erosivity factor (R)	23

3.5.4.. Soil Erodibility Factor (K)	25

3.5.5. Topographic (LS) Factor	26

3.5.6. Biophysical CSV Table	27

3.5.7. Land Use/Land Cover Management Factor (C-Factor)	27

3.5.8. Support Practice Factor (P-Factor)	29

3.6. Model Structure	30

3.7. Annual Soil Erosion	31

3.8. Annual Sediment Export	31

3.9. Methods of Data Analysis	33

3.10. Data Validation	33

4. REULTS AND DISCUSSIONS	36

4.1. LULC Changes in the Study Watershed from 1990 to 2020	36

4.1.1. Accuracy of the Classified Images	38

4.2. Land Use Land Cover Change Detection	39

4.3. Analyses of LULC Conversions	40

4.3.1. Conversions between 1990 and 2000	41

4.3.2. Conversions between 2000 and 2010	41

4.3.3. Conversions between 2010 and 2020	42

4.4. Soil erosion and sediment export in the maybar watershed	43

4.4.1. Soil erosion	43

4.4.2. Sediment export	47

4.5. Validation of model performance	49

5. CONCLUSION AND RECOMMENDATION	51

5.1. Conclusion	51

5.2. Recommendation	51

6. REFERANCE	53

# LIST OF TABLES

Table3.1. The satellite image description used for this study	20

Table3.2. LULC classes’ description (adopted from Gebremedhin Tesfaye et al., 2017)	21

Table3.3. The mean annual rainfall and corresponding R-factor	24

Table3.4. The soil types and the corresponding K-values recommended by Hurni (1985)	26

Table3.5. Adopted values of RUSLE C_ factor for different LULC classes	28

Table3.6. Adopted values of RUSLE P-factor for different LULC classes	30

Table3.7. Objective functions and model performance ratings for simulations of sediment yield.	35

Table4.1. Land use land cover classification of 1990; 2000; 2010 and 2020 years	38

Table4 2. Accuracy assessment in percent % of lulc change map of each year	39

Table4.3. Land use and land-cover change detection	40

Table4.4. Transition area matrix in hectare between 1990 to 2000 periods in the maybar watershed	41

Table4.5. Transition area matrix in hectare between 2000 to 2010 periods in the maybar watershed	42

Table4.6. Transition area matrix in hectare between 2010 to 2020 periods in the maybar watershed	43

Table4.7. The total soil erosion and mean soil erosion rated of entire watershed during 1990-2020 periods.	45

Table4.8. The contributions of eache LULC type to the total soil loss in the study watershed during 1990-2020 periods.	46

Table4.9. The total sediment export and mean sediment export of entire watershed during 1990-2020 periods.	48

Table4.10. The contributions of eache LULC type to the total sediment export in the study watershed during 1990-2020 periods.	49

Table4.11. Model performance statistics for the validation periods of 1990 to 2020	50

# LIST OF FIGURES

Figure 3 1. Location map of study area	19

Figure 3. 2. Location of rain gauge stations in and around the study watershed	24

Figure 3 .3. The R-factor maps of the study watershed	25

Figure 3. 4. Soil types (a) and K-factor (b) maps of the study watershed	26

Figure 3 .5. The slope (in degree) (a) and LS-factor (b) maps of the study watershed	27

Figure 3. 6. The C-factor maps of the watershed in 1990 (a), 2000 (b), 2010 (c) and 2020(d) years	29

Figure 3 .7. The P-factor maps of the study watershed	30

Figure 4. 1. Land use land cover classification map of 1990; 2000; 2010 and 2020 years	37

Figure 4. 2. Land use land cover	38

Figure 4. 3. Land use land cover detection	40

Figure 4. 4. Soil loss map of each land use t/ha/year	45

Figure 4. 5. Sediment export map of each land use t/ha/year	48

# 1. INTRODUCTION

## 1.1. Background

Soil loss is a key environmental challenge facing the world (Jahun et al., 2015; Maliqi and Singh, 2019). It is an ecological process of detachment, transportation, and deposition of soil materials by erosive agents, mainly rainfall, wind, and gravity (Aksoy et al., 2019; Negessa Gadisa and Tesfaye Midega 2021). Soil loss and sediment export are direct products of the complex interactions between natural (e.g., rainfall, topography, soil characteristics, etc.) and anthropogenic factors such as agricultural land use, deforestation, and urbanization (Phinzi and Ngetar, 2019).

Generally, soil loss and sediment export negatively affect ecosystem services and functions (Arega Degife et al., 2021; Hamere, Yohannes et al., 2021). It aggravates rocky desertification (Guo et al., 2022). Soil loss affects agricultural productivity (Kebede, Wolka et al., 2015). That affects the rural population's food and nutrition security (Arega Degife et al., 2021; Nigussie Haregeweyn et al., 2017; Belayneh Yigez et al., 2021). It also causes environmental and socio-economic problems due to sediment load in the downstream area. For example, the deposition in the water body can increase the death of aquatic life and water quality pollution. In addition, increasing the cost of managing the water body, shortening the water reservoir’s life span, and flood occurrence can be other examples of socio-economic problems due to the deposition (Belayneh Yigez et al., 2021).

Ethiopia is one of the sub-Saharan countries that experience severe soil erosion, with an average annual loss of 16–300 t/ha/year in cultivated lands (Hurni, H., 1988). It results in food insecurity and a loss of 2–3% of agricultural GDP (Kirui and Mirzabaev, 2009). Factors such as Slope steepness, long cultivation history with outdated technology, and overgrazing make soil erosion more severe (Basin et al., 2019).

The annual sediment load from agricultural landscapes is similarly worrisome for soil loss in the highlands of Ethiopia (Setegn Shimelis et al., 2010). Because the water quality and storage potential of natural lakes in the country has deteriorated significantly from a large number of sediment loads from the upland areas (Brook Lemma, 2004; Fekadu Yohannes, 2005; Setegn Shimelis et al., 2010). Due to uncontrolled erosion and sedimentation from the surrounding agricultural landscapes Lake Alemaya in the eastern highlands has already dried out (Brook Lemma, 2004; Fekadu Yohannes, 2005; Eshetu Tufa et al., 2014). In the Central Rift Valley area other lakes like Abijata can soon dry out (Derege Tsegaye et al., 2012; Habtamu Temesgen et al., 2013).

The modification of Earth’s terrestrial surface by human activities is commonly known as land use/land cover change around the globe (Agumassie Genet., 2020; Liaqat et al. 2021). Land use and land cover change have two separate terminologies that are often used interchangeably (Rawat and Kumar 2015; Rasool et al. 2021). Land cover refers to the biophysical characteristics of the earth’s surface, including the distribution of vegetation, water, soil, and other physical features of the land (Delalay et al. 2019; Wang et al. 2018). While land use refers to how the land has been used by humans and their habitat, usually with an emphasis on the functional role of land for economic activities (Lambin and Linderman 2006; Magliocca et al. 2015).

In Ethiopia LULC change is mainly dominated by the conversion of natural vegetation cover to use for agriculture activities (Woldeamlak Bewket, and Solomon Abebe, 2013, Temesegen Gashaw et al., 2018). According to FAO, (2020), the forest cover in Ethiopia has decreased from 13.3% of the total area of the country (14.69 million ha) in 1993 to 11.4% of the total area (12.54 million ha) in 2016 with an estimated annual rate of change 0.8% (104,600 ha/year). On the contrary, the agricultural land of the country has increased from 27.66% of the total area of the country (30.54 million ha) in 1993 to 32.83% of the total area (36.26 million ha) in 2016. Due to early settlement and the resulting population pressure, however, LULC changes were much pronounced in the highlands of Ethiopia (Solomon Gebreyohannis et al., 2010; Rientjes et al., 2011; Gebremicael et al., 2013).

## 1.2. Statement of the Problem

Many studies reported the effect of soil erosion on agricultural land and water resources. For instance, Tully et al., (2015) reported that more than two third of cropland degradation and the resulting productivity loss in Africa resulted from soil erosion. Besides, Ahmed Moustafa (2018) reported that Aswan High Dam of Egypt has lost 4% of its water storage capacity over 48 years; Khashm el-Girba reservoir of Sudan has lost 53% of its water storage capacity in 46 years; Sennar reservoir of Sudan has lost 85% of its water storage capacity in 85 years. Moreover Arega Degife et al., (2019) revealed that the loss of Lake Cheleleka and the degradation of the surrounding wetlands in the Central Rift Valley region of Ethiopia which was resulted from the erosion and deposition of sediment from the surrounding farmlands and Angereb reservoir in Ethiopia has lost 46% of its water storage capacity in 19 years due to problems of sedimentation.

A quantified estimation of soil erosion and sediment export and identification of the sources of sediments are great interest in water resources management (Ambers 2001; Navas et al., 2009; Sharp et al., 2018). It helps to address the problem through proper planning and allows the design of better strategies for reducing the impacts on downstream irrigation, water treatment, recreation and reservoir performance (Sharp et al., 2018).

Literature indicates that not all of the eroded soil in a watershed is transported out of it a significant proportion of the eroded soil gets deposited at intermediate sites (Constable and Belshaw, 1986; Swarnkar et al., 2018). A study by Constable and Belshaw (1986) revealed that only 10% of the total soil erosion is delivered to the watershed’s outlet and reaches a natural water body. This exported soil offers the downstream influences of sediment deposition affecting the ecosystem and siltation of lakes and hydroelectric dams (Baral et al., 2014). Hence, it is important to quantify and differentiate between the level of soil erosion (the amount of soil eroded in a watershed) and sediment export (i.e. the proportion of soil delivered to the stream that reached a watershed’s outlet), in a watershed to identify the level of downstream impacts for effective water resources management (Sharp et al., 2018).

Although there were many studies on soil loss and sediment yield at the global level, most of them focused on the use of sophisticated instruments and well experienced experts in data rich environments (Jansson 1988; Syvitski and Milliman 2007). Such approaches are less practical in the context of developing countries including Ethiopia, where there are data scarcity and a lack of experienced experts (Nigussie Haregeweyn et al., 2012). Studies in developing countries on soil erosion widely employ the Revised Universal Soil Loss Equation (RUSLE) to estimate soil loss (Hamel et al. 2015). However the equation is not capable of estimating the sediment export of a given watershed (DeVente et al., 2013). Since it doesn’t determine sediment pathways from steep slopes to water bodies, it is difficult to examine possible downstream impacts, such as pollution and sedimentation (Jahun et al., 2015). The Integrated Valuation of Ecosystem Services and Tradeoffs (InVEST) sediment delivery ratio (SDR) model has the advantage of addressing the limitations of the RUSLE thus enabling the combined estimations of soil loss and sediment export through characterizing the hydrological connectivity of a given watershed (Sharp et al., 2018). Hence this study employs the InVEST SDR model and seeks to utilize the advantages of the model’s comprehensive estimation of soil loss and sediment export, of the watershed.

However, the levels of erosion and sediment yield reported show spatial variation depending on the type of soil, climate, topography, population density, farming, management practices, land use land cover and methods used. Such variations signify that site-specific studies and locally adaptable erosion and sediment mitigation strategies are still necessary to minimize the impacts of accelerated erosion and sedimentation. Additionally, although the studies indicate soil erosion and related sediment yield in the country have led to various environmental problems, there were very limited studies on the estimations of erosion and sediment yield in the maybar watershed. Moreover, studies that integrate the estimations of soil loss and sediment export along with studying their effect on  land use land cover are still scanty, if not unavailable for the maybar watershed  in particular with the use of InVEST  SDR  model.

Therefore, the present study was conducted in Maybar watershed the northeastern highland region of Ethiopia. Previously, above all very limited studies in Ethiopia and probably there is no study carried out in the maybar watershed  with respect to effects of  LULC changes on soil erosion  and sediment yield employing with InVEST SDR model.. Hence, the purpose of this study was to quantify and map soil erosion and sediment export, along with the effects of land use land cover change in the study Watershed. This will help to understand the level of stress on natural resources and make informed decisions before irreversible damages of the watershed resources and use as a guide line for police makers.

## 1.3. Objectives of Study

### 1.3.1. General objective

The general objective of this study is to model the effects of LULC change on soil erosion and sediment yield in the maybar watershed using InVEST model

### 1.3.2. Specific Objective

- To analyze the LULC changes from 1990 to 2020.

- To estimate soil erosion and sediment yield from 1990 to 2020 due to LULC changes.

## 1.4. Research Questions

Based on the stated objectives, the following questions have been used to guide the research process and finally answered from the findings of the study.

i. What is the LULC change of the Maybar watershe in the period of 1990 to 2020?

ii. What is the mean annual soil erosion in Maybar watershed from 1990 to 2020?

iii. What is the mean sediment yield in Maybar watershed from 1990 to 2020?

## 1.5. Scope of the Study

The scope of this study is focuses mainly on effects of lsnd use land cover change on soil erosion and sediment yield in Maybar watershed in the north eastern highland of Ethiopia between the analysis period of 1990 to 2020.

## 1.6. Limitation of the Study

This study encountered certain challenges. One of the challenges the current national crisis of war, lack of daily recorded sediment data, time and money also limit research.

## 1.7. Significance of the Study

The study will have its own rationalities both for study site in one way and for LULC change literatures. By analyzing the LULC change trend and driving forces behind the changes, it helps to understand how land was being used in the past. Moreover it can provide data to policy and decision makers to design appropriate policies and strategies and promote sustainable management of natural resources. More sustainable management of natural resources in turn can enhance agricultural productivity. A large number of government or non-government agencies, researchers and local communities can benefit from the outputs of this research.

# 2. LITERATURE REVIEW

## 2.1. Soil Erosion

Soil erosion is defined as the wearing away of the land surface by physical forces such as rainfall, flowing water , wind, ice, temperature change, gravity or other natural or anthropogenic agents that abrade, detach and remove soil or geological material from one point on the earth surface to be deposited elsewhere. In the long term, this process leads to stable landforms with low erosion rates (Haile Mitiku et al., 2006). There are two main types of erosion: geologic and accelerated erosion. Geologic erosion is a normal process of weathering that generally occurs at low rates in all soils as part of the natural soil-forming processes. It occurs over long geologic time horizons and is not influenced by human activity. In contrast, soil erosion becomes a major concern when the rate of erosion exceeds a certain threshold level and becomes rapid, known as accelerated erosion. This type of erosion is triggered by anthropogenic causes such as deforestation, slash-and burn agriculture, intensive plowing, intensive and uncontrolled grazing, and biomass burning (Blanco and Lal, 2008).

Soil erosion is a two-phase process consisting of the detachment of ind ividual soil particles from the soil mass and their transport by erosive agents such as running water and wind. When sufficient energy is no longer available to transport the particles, a third phase, deposition, occurs (Morgan, 2009). Soil erosion affects the physical and chemical properties of soils. The physical parameters are primarily organic matter content, structure, texture, bulk density, infiltration rate, rooting depth, and water-holding capacity. Changes in chemical parameters are largely a function of changes in physical composition. Loss of soil implies loss of productive land, as a consequence it reduce the productivity of agricultural land and crops yield which is actually essential to sustain human life on earth. However, in areas where the topsoil is acid and the organic matter content is initially low, surface erosion may, in fact, increase crop yields due to the exposure of more favorable subsoil (Haile Mitiku et al., 2002).

### 2.1.1. Global Context of Soil Erosion

One third of the world’s agricultural soils, or roughly 2 billion hectares of land, was reported as being affected by soil degradation. Water and wind erosion account for 84% of this observed damage (Hurni, 2002). Water erosion representing about 56% of the total degraded land and wind erosion affects about 28% of the total degraded land (Blanco and Lal, 2008). While other forms like physical and chemical degradation are responsible for the rest. Industrialisation and urbanisation have further caused specific forms of soil degradation. Most damage, however, is the result of inappropriate land management in all the diverse farming systems, from subsistence to mechanized farming (Hurni, 2002). Thus, water and wind erosion are the dominant agents that degrade soils (Blanco and Lal, 2008). The problem of soil erosion is severe particularly in the tropics and sub-tropics because of the high population pressure, scarcity of prime agricultural lands, and predominance of resource-poor farmers. It is much severs in Africa and Asia (Blanco and Lal, 2008).

One regional base, measurements of soil loss undertaken from hill slopes in West Africa, ranging in steepness from 0.3° to 4°, yield annual rates of 0.15, 0.20 and 0.03 t/ha/year under natural conditions of open savanna grassland, dense savanna grassland and tropical rain forest respectively. Clearance of the land for agriculture increases the rates to 8, 26 and 90 t/ha/year, while leaving the land as bare soil produces rates of 20, 30 and 170 t/ha/ year respectively. Thus, removal of the rain forest results in much greater rises in erosion rates than does removal of the savanna grassland. These measurements emphasize the high degree of protection afforded by the rain forest but also reflect the erosive capacity of the high rainfalls in the humid tropics when that protection is destroyed. The rates of removal of tropical rain forests over the past twenty years are therefore of major concern with respect to present and future erosion problems (Roose 1971 cited in Morgan, 2009). Similar with the above finding, (Schwab et al., 1993) have described the function of vegetation in erosion control on a more comprehensive way. According to them, the major effects of vegetation in reducing erosion are interception of rainfall by absorbing the energy of the raindrops and thus reducing surface sealing and runoff, retardation of erosion by decreased surface velocity, physical restraint of soil movement, improvement of aggregation and porosity of the soil by roots and plant residue, increased biological activity in the soil and transpiration , which decrease soil water, resulting in increased storage capacity and less runoff. Due to the variations of environmental factors and conservation efforts, the amount of soil loss also varies among countries. In addition, soil loss also varies in different land use/land cover classes. It is very high in cultivated land and bare soil than under natural conditions. Similarly, studies indicated that land use/land cover changes towards cultivated and degraded land increases the susceptible of the soil for erosion (Gete Zeleke and Hurni H 2001; Woldeamlak Bewket, 2002).

### 2.1.2. Soil Erosion in Ethiopia

Soil erosion is recognized as one of the most serious causes of soil degradation in Ethiopia (Awdenegest Moges and Holden, N.M., 2007, Menale Kassie, 2011, Woldeamlak, Bewket, and Sterk, G., 2002) and hence in highland areas of the country the crop yield and soil fertility levels are extremely low (Woldeamlak, Bewket, and Sterk, G., 2002) Continued soil erosion seriously threatens peoples’ livelihoods, especially in highland parts of the country, where arable land is a very scarce resource (Woldeamlak, Bewket, and Sterk, G., 2002). Annually about 2 billion cubic meters top soil loss is reported for the country (Hurni, 2015, Hurni, 1989, and Woldeamlak, Bewket, and Sterk, G., 2002). According to (Hurni, 1985) the average annual soil erosion rate in Ethiopia was 18t/ha/year. Ethiopia losses over 1.5 billion tons of top soil annually from the highland area due to erosion (Ermias Teferi, 2016), The average annual soil loss rates from crop lands estimated as 42 t/ha/year and ranged up to 300 t/ha/year in extreme cases (Hurni, 1993). Especially the productive highland parts of the country, which characterized by rugged topography, densely populated, intensive cultivation and that host large proportion of people and livestock, harmed highly with problem of land degradation(Bekele Tsegaye., 2019 Mushir A, Kedru S 2012, Tegegne Molla and (Biniam Sisheber.,2017). About 43% of the total highland areas of Ethiopia are highly affected by soil erosion with an estimated average annual loss of 20 t/ha/year and measured annual amounts of more than 300t/ha/year on specific plots (Hurni H 1990, Dubale, P., 2001). Not only the highlands, but also the dry lowland areas of the country have also been identified as vulnerable to the problem of land degradation (UNEP, 2013).

## 2.2. Land Use Land Cover Change

The relationship between land use and land cover can be described as: change in land use can affect and be affected by land cover, however the change in either of them is not necessarily the product of the other. Single land use system may correspond to a single land cover or it may involve several distinct covers (Briassoulis, 2011). For instance a farming system may involve several distinct covers such as cultivated land, woodlots, improved pasture, and settlements. On the other hand single class of cover may support multiple uses. For example the area covered by natural forest can be used for hunting and gathering, fuel wood collection, recreation and wild life preservation (Briassoulis, 2011; Verheye, 2009)

A change in land cover refers to conversion of one land cover type to a new cover type or modification within one land cover category (Lambin and Geist, 2003). On the other hand land use change refers to a conversion of land use due to the interference of human being for different purposes such as for settlement, infrastructural development, agriculture and recreational uses (Turner et al., 1995). LULC change refers to the human modification and conversion of the earth terrestrial surface (Shrestha, 2012). Modification occurs when the change affect only the characteristics of the land cover without causing a complete shift from one LULC type to the other. On the other hand conversion of LULC occurs when one LULC type completely replaced by another (Binyam Alemu et al., 2015)

### 2.2.1. Land Use Land Cover Change at Global Perspective

LULC Change occurs at local, regional and global scales and changes at local scales can have cumulative impacts at broader scales (Ashenafi Burka., 2008). LULC Change is as old as the age of human kinds (Hagos Gebreslassie., 2014). Human land use activities spread over about 50% of the ice free land surface starting from the control over fire and domestication of animals and plants (Lambin and Geist, 2003). The spread of human land use activities were mainly at the expance of forest lands, resulting reduction of global forest cover from 50% to less than 30% (Lambin et al., 2003). Between 1700 and 1980 for instance, at the global scale total cultivated land was estimated to have increased by 46% (Turner et al., 1992 Cited in Muluneh, 2003). Furthermore, between the years 1700 and 1990, the area under cropland and pasture has increased from an estimated 300- 400 million ha to 1500-1800 million ha, and 500 million ha to 3100 million ha respectively. This resulted in reduction of forests from around 5000-6200 million ha in 1700 to 4300-5300 million ha in 1990 and natural grasslands, steppes and savannas from around 3200 million ha in 1700 to 1800- 2700 million ha in 1990 (Lambin et al., 2003).

However LULC Change is not homogeneous across all parts of the world (WIREs Climate Change, 2014). Some regions experienced huge LULC Change mainly expansion of agricultural lands at the expanse of vegetated lands. This is true in regions with a shorter history of human development such as Africa, south and Southeast Asia and Latin America countries. Since 1850, these regions have experienced dramatic increases in cropland, especially during the second half of the twentieth century (Lambin et al., 2003; WIREs Climate Change, 2014). On the other hand expansion of agricultural lands reduced in European countries (WIREs Climate Change, 2014).

### 2.2.2. Land Use and Land Cover Change in Ethiopia

LULC change is the human modification of Earth’s terrestrial surface from existing management of the land or land cover to new management of land or new land cover type (Sisay Nuru et al., 2016). The nature of LULC dynamics significantly differs from country to country as the drivers of LULC change and land management activities vary from place to place.

In Ethiopia, LULC change is mainly dominated by the conversion of natural vegetation cover to use for agriculture activities (Woldeamlak Bewket and Solomon Abebe., 2013, Temesgen, Gashaw et al., 2018). According to FAO (2020), the forest cover in Ethiopia has decreased from 13.3% of the total area of the country (14.69 million ha) in 1993 to 11.4% of the total area (12.54 million ha) in 2016 with an estimated annual rate of change 0.8% (104,600 ha/year). On the contrary, the agricultural land of the country has increased from 27.66% of the total area of the country (30.54 million ha) in 1993 to 32.83% of the total area (36.26 million ha) in 2016.

Several research works (Asnake Yimam and Amare Bantider., 2019, Adenew Taffa et al., 2015, Woldeamlak, Bewket, and Solomon Abebe., 2013, Mesfin Anteneh et al., 2016, Binyam Alemu, et al., 2015, Kiros Tsegay and Desalegn Wana.,2019, Wakjira Takala et al., 2020, Sisay Nune et al., 2016,  BelayTegene., 2002, and Diress Tsegaye et al., 2010) were undertaken on the spatial and temporal land use and land cover change at watershed and district level in Ethiopia and have shown that there has been a significant LULC change in different parts of the country dominated by the expansion of cultivated land and built-up areas at the expense of natural vegetation cover, shrub lands and grass lands particularly in the highlands

A study by Woldeamlak, Bewket, and Solomon Abebe., (2013) reported the consistent expansion of cropland and settlement at the expense of forest and dense tree cover over the period between 1957 and 2001 in Gish Abay Watershed, Blue Nile Basin of Ethiopia. A similar study by Ebrahim Esa Hassen and Mohamed Assen., (2018) reported that farmlands and settlement areas were expanded at the expense of forest area, shrub lands, and grasslands over the study period between 1957 and 2014 in Gelda catchment, Lake Tana Watershed of Ethiopia. Samuale Tesfaye et al., (2014) reported a significant increase in cultivated land and settlement and a decrease in vegetation cover between 1976 and 2008 in Gilgel Tekeze Catchment, Northern Ethiopian Highlands. According to Alemayehu A Shawul, and Chakma, S., (2019), cropland and urban area replaced a large area of forest cover and shrub lands in Upper Awash Basin, Ethiopia, over the years between 1972 and 2014. Wakjira Takala et al., (2020) also found an expansion of agricultural land and built-up area at the expense of forest land in Finchaa Catchment, Northwestern Ethiopia, over the period between 1987 and 2017 Similarly, Abreham Berta et al., (2020) reported the expansion of cultivated land, built-up area, and bare land at the expense of forest land and shrubland between 1988 and 2018 in Winike Watershed of Omo Gibe Basin, Southwest Ethiopia. A study conducted by Fikirte Demissie et al., (2017) also reported the continuous expansion of cultivated lands at the expense of forest area and grasslands over 42 years (1973–2015) in Libo Kemkem District of Northwestern Ethiopia. An increase in farmland area at the expense of forest land was also observed in Bale Mountain Eco-Region of Ethiopia from 1985 to 2015 (Sisay Nune et al., 2016). Another study by Mesfin Anteneh et al., (2016) found a significant increase in cultivated land and settlement and a decrease in forest land, shrubland, grassland, and wetland over 48 years (1957–2005) in Gumara Watershed of Lake Tana basin, Northwestern Ethiopia. Asnake Yimam and Amare Bantider., (2019) reported the consistent increase in farmland and settlement area at the expense of Afro-alpine and sub-Afro-alpine vegetation areas between 1973 and 2017 in Beshillo Catchment of Blue Nile Basin, Northeastern Highlands of Ethiopia

A study conducted by Kiros Tsegay and Desalegn Wana, (2019) revealed that forest area was converted into agricultural land and barren land between 1957 and 2017 in the Central Highlands of Ethiopia. Girma Ayele et al., (2019) also found that farmland expansion was the major cause for the reduction of forest area between 2000 and 2015 in Delomena District, Bale Zone of Ethiopia. Belay Tegene. (2002) reported a significant conversion from natural vegetation cover to cropland was observed between 1957 and 1986 in Derekolli Catchment of the South Wollo Zone of Ethiopia

The expansion of farmlands at the expense of other land use and land cover types has occurred not only in the highlands of Ethiopia but also in lowland arid and semiarid areas of the country. According to Binyam Alemu et al., (2015), the expansion of agricultural land and bare land and the reuction of woodland cover were observed between 1985 and 2010 in the Northwestern lowlands of Ethiopia. Tsegaye Diress et al., (2010) also found a rapid reduction in the size of woodland and grassland cover, and more than the eightfold increment in the size of cultivated land took place between 1972 and 2007 in arid and semiarid Northern Afar range lands of Ethiopia. Hagos Gebreslassie, (2014) reported cultivated lands and open lands have shown continuous expansion at the expense of forest lands, shrub lands, and grasslands in Huluka Watershed, Central Rift Valley of Ethiopia, between 1973 and 2009. Cropland expanded at the expense of forest, woodlands, and grasslands between 1973 and 2014 in the Central Rift Valley of Ethiopia (Adenew Taffa et al., 2015). In the Northeastern Somali Rangelands of Ethiopia, grassland cover was converted into cultivated and settlement areas between 1985 and 2017 (Fikre Zerfu et al., 2019)

In Contrary to the findings of the aforementioned studies in different parts of the country, Amare Bantider et al., (2011) reported the expansion of forest area and shrub land and the decline of the area of cropland since the mid-1980s in the Eastern Escarpment of Wollo, Northeastern Ethiopia. Asmamaw L. B et al., (2011) have also shown the expansion of forest land during the study period between 1958 and 2006 in Gerado Catchment of Northeastern Ethiopia.

## 2.3. Sediment Yield

The quantity of eroded material carried from a plot, area, channel, or watershed is referred to as sediment yield (Renard et al., 1997). As a result, soil erosion is the primary source of sediment yield, and no sediment yield to any given point or stream channel occurs unless soil erosion has occurred over time (Megersa Kebede and Chakravarti, 2017). All of the soil erosion that occurred on the upper slopes, however, did not result in sediment yield reaching a downstream point  This is because when the erosive agent has no sufficient energy to transport the eroded soil particles, some of them are deposited in the meantime before reaching the down-stream point (Morgan, 2009). Sedimentation, the deposition of sediment particles after separating from their origins, reduces the storage capacity of lakes and reservoirs. Therefore, higher sediment yield and ultimately, reservoir and lake sedimentation are indicators of a higher soil erosion rate in the upland areas. Hence, it is apparent that sediment yield is dependent on soil erosion, and Sedimentation depends on the yield of sediment

The annual sediment load from agricultural landscapes is similarly worrisome for soil loss in the highlands of Ethiopia (Setegn Shimelis et al., 2010). Because the water quality and storage potential of natural lakes in the country has deteriorated significantly from a large number of sediment loads from the upland areas (Brook Lemma, 2004; Fekadu Yohannes, 2005; Setegn Shimelis et al., 2010). Due to uncontrolled erosion and sedimentation from the surrounding agricultural landscapes, Lake Alemaya in the eastern highlands has already dried out (Brook Lemma, 2004); Fekadu Yohannes, 2005; Eshetu Tufa et al., 2014). In the Central Rift Valley area, other lakes like Abijata can soon dry out (Derege Tsegaye et al., 2012; Habtamu Temesgen et al., 2013). Besides, a large amount of sediment loads from un-managed uplands has threatened water supply and power generation reservoirs in the country. Good examples of affected power generation reservoirs are Koka, Gilgel Gibe I, Aba Samuel, MelkaWakena; and water supply reservoirs are Angereb, Legedadi, Borkena, and Adrako as well as several irrigation reservoirs in the northern highlands (Kebede Wolka, 2012).

## 2.4. Description of Soil Erosion Models

### 2.4.1. Invest SDR Model

The Integrated Valuation of Ecosystem Services and Tradeoffs (InVEST) and Sediment Delivery Ratio model is used to quantify and map sediment delivery and soil loss in a land escape (Hamel et al., 2017; Sharp et al., 2018). The main reason for using this model includes: the model is comparatively less data-intensive and more flexible than other models such as SWAT, it can be easily adapted to a particular context and to either global or local data readily available (Gao et al,2017 ), and it shows the hydrological connectivity developed by (Borselli et al., 2012). The unique characteristic of the InVEST SDR model is its ability to evaluate soil erosion and sediment export from each type of land use and to measure the amounts of sediment, including streams and reservoirs, joined to water bodies.

### 2.4.2. Revised Universal Soil Loss Equation (RUSLE) Model

The Revised Soil Loss Equation (RUSLE) model (Renard et al., 1996 & 1997) is often used to estimate soil erosion rate. RUSLE is an extension of the Universal Soil Loss Equation (USLE) model by adapting the input factors to the local conditions Renard et al., 1996). Because of its clear and relatively simple computational inputs, RUSLE has been widely applied in Ethiopia (e.g. Woldeamlak, Bewket and Ermias Teferi, 2009; Abate Shiferaw, 2011a; Derege Tsegaye et al., 2012) and elsewhere in the world (e.g. Angima et al., 2003 in Kenya; Yue-Qing et al., 2008 and Xu et al., 2012 in China; in the United States; Prasannakumar et al., 2012 in India; Lulseged Tamene, and Quang Bao Le., 2015 in Sub Saharan Africa). RUSLE (Renard et al., 1997) and its predecessor USLE (Wischmeier and Smith, 1978) estimate average annual soil loss (A) from sheet and rill erosion as a function of five factors

A=R*K*LS*C*P------------------------------------------------------------ 1

Where, R, K, LS, C and P are rainfall erosivity, soil erodibility, topographic, cropping and landcover and erosion control practice factors, respectively

Rainfall Erosivity (R) factor

The R-factor accounts the erosive force of a specific rainfall (Wischmeier and Smith, 1978; Prasannakumar et al., 2012; Alexakis et al., 2013), which is determined by the amount, intensity and distribution of rainfall (Tadesse, Amsalu, and Abebe Mengaw., 2014), of which intensity is the most rainfall property that determines the amount of erosion .Hence, in the original USLE and its revised version (RUSLE), the R-factor was represented in the rainfall intensity data (Wischmeier and Smith, 1978; Woldeamlak, Bewket and Ermias Teferi., 2009; Kouli et al., 2009; Syed Ahmad,Ali and Hamelmal Hagos., 2016).

Soil Erodablity (K) factors

The K-factor corresponds to the influence of the soil physical and chemical properties on erosion during storm events in upland areas (Wischmeier and Smith, 1978; Renard et al., 1996; Ali, Syed Ahmad, and Hamelmal Hagos.,2016). Few soil properties which affect soil erodiblity include soil texture, drainage condition, soil depth, structural integrity and organic content (Gebreyesus, Brhane, and Kirubel Mekonen, 2009; Prasannakumar et al., 2012). Among the various methods of determining the K-factor, the soil nomograph, which uses the relative proportions of soil texture, permeability, soil structure and organic matter content (Wischmeier and Smith, 1978; Abate, Shiferaw, 2011a; Hailu Kendie and Klik, 2015; Kebede Wolka et al., 2015), is the most commonly used method.

Topographic (LS) factor

The LS-factor account the influence of topography on erosion and it includes both the slope length (L-factor) and slope steepness (S-factor) factors (Wischmeier and Smith, 1978; De Asis and Omasa, 2007; Kouli et al., 2009; Prasannakumar et al., 2012). In the steeper and longer slopes, erosion by water is greater due to the greater influence of topography in increasing runoff (Wischmeier and Smith, 1978; Abate Shiferaw, 2011a; Alexakis et al., 2013). Conversely, in gentle and shorter slopes, soil erosion by water is lower. Hence, the LS-factor in the model is one of the major surface parameters for erosion (Alexakis et al., 2013).

Cropping and Land-Cover (C) factor

The C-factor accounts the role of vegetation covers on soil erosion (Wischmeier and Smith, 1978; Morgan, 2005). In areas where vegetation is absent soil erosion by water is higher. In reverse, soil erosion on the vegetated land is lower, which is due to the higher protection of the soil surface by vegetation against erosion (Alexakis et al., 2013). Therefore, one can reduce soil erosion by reverting the LULC types into the directions of having more vegetated covers (Temesgen Gashaw et al., 2017). Because of this, the C-factor is perhaps the most important factor to reduce soil erosion.

Erosion control practice (P) factor

The P-factor accounts the role of conservation practices in reducing erosion (Wischmeier and Smith, 1978). The value of P-factor is between 0 and 1. In general, higher values are assigned to areas with no conservation practices while minimum values approaching 0 are provided for areas with good conservation practices (Wischmeier and Smith, 1978; Prasannakumar et al., 2012; Ganasri and Ramesh, 2016). Therefore, the lower the P value is, the more effective is the practice of protecting erosion (Prasannakumar et al., 2012; Ganasri and Ramesh, 2016).

# 3. MATERIALS AND METHODS

## 3.1. Study Area Description

The Maybar watershed is located in the Amhara National Regional State, South Wollo Zone, dessie zurie district, Ethiopia. It is situated at about 20 km south east of the city of Dessie and about 422 km north of Addis Ababa. It was the first research site established by Soil Conservation Research Project’s (SCRP) in 1981. Geographically, the study site lies at 39°39ʹ0ʹʹ to 39°40ʹ0ʹʹ  E and 11°0ʹ0ʹʹ to 11°0ʹ30ʹʹ N and it is characterized by highly rugged topography with steep slopes ranging between 2481 and 2814 m above sea level with an area of 116.27 ha (Bosshart, 1997). It is categorized under the moist Weyna Dega/Moist Dega agroclimatic zone. The study area is characterized as an erosion prone, low potential and oxen-plowed cereal belt of the north-eastern escarpment of the Ethiopian highlands (SCRP, 1982).

Slopes range from 0% to 49.7%. Rainfall is characterized by a bimodal pattern with an erratic distribution with the mean annual rainfall of 1448 mm (Haile Mitiku et al., 2006). The small rainy season (Belg) occurs from March to May, where as the main rainy season (Kiremt) occurs from June to September and the dry season from October to February. The area has sub-humid with annual mean minimum and mean maximum temperatures of 11.43°C and 21.6°C, respectively.

The soil is dominated by Eutric Leptosols 71.4%, Eutric Regosol 19.1% and Eutric Vertisols 9.5% of watershed area coverage (SCRP, 2000). A mixed farming system such as livestock husbandry and crop production is the farming system in the Maybar watershed with rainfed agriculture. Soil conservation interventions such as level soil and stone bunds were introduced in the catchment in 1983 through a Food for Work campaign conducted by the Ministry of Agriculture (SCRP, 2000).

Figure 3 1. Location map of study area

## 3.2. Data Types, Sources and Analysis

### 3.2.1. Satellite data source and processing

Four landsat images were acquired from the http://www.earthexplorer.usgs.gov website using Landsat 4–5 TM 1990, Landsat 7 ETM+ 2000, Landsat 7 ETM+ 2010 and Landsat 8 OLI/TIRS 2020 (Table 1). Analysis of Landsat images for the period 1990–2020 with equal year interval (1990, 2000, 2010, and 2020).

Ground truth data were taken by GPS and Google earth to develop the training sites. These points were overlaid onto true composite images for the delineation of the training areas. Each training area contained at least one pixel that was no more than two pixels away from the original ground truth point. It must be stressed to the original ground truth pixels.

Composites of various band combinations were used to assist in the allocation of training regions for specific features. For instance, forest training sites were digitized over a (RGB) 4, 3, 2 bands combination because forest vegetation appeared red to bright red in the false color composite image and green to dark green in the true color composite and (RGB) 4, 5, 3 (RGB) 7, 4, 2 and (RGB) 5, 4, 1 also used which its better suited in visualizing agricultural, vegetation and other lands (Http: //gif. berkeley.edu., 2003).

Table3.1. The satellite image description used for this study

| Satellite data (image) | Path and raw | Date of acquisition | Sensor | Number of bands | Spatial resolution (meter) | Cloud cover |
| --- | --- | --- | --- | --- | --- | --- |
| Landsat 8 | 168 and 52 | January 19/1/2020 | Operational land imager /thermal infrared sensor OLI/TIRS | 11 | 30 | 0 |
| Landsat 7 | 168 and 52 | January 15/1/2010 | Enhanced thematic mapper plus image ETM+ | 8 | 30 | 0 |
| Landsat 7 | 168 and 52 | February 5/2/2000 | Enhanced thematic mapper plus image ETM+ | 8 | 30 | 0 |
| Landsat 4-5 | 168 and 52 | February 17/2/1990 | Thematic mapper (TM) | 7 | 30 | 0 |

## 3.3. Image Classification Methods

Preprocessing tasks such as geometric and radiometric corrections and geo-referenced to WGS 1984 UTM projection was applide before classifying the images. The type of radiometric corrections undertaken was histogram equalization, haze reduction, noise reduction, etc. For land cover classification the supervised classification method in ArcGIS 10.3.1 software using the decision rule of maximum likelihood classifier algorithm was used. Supervised classification was done making use of ground checkpoints and satellite images of the study area.

Training area selection was an important step in the classification which could be homogeneous and represent LULC classes was determined. Training data were collected from field (ground survey) for 2020 image data, and for 2010 image data, collected from Google earth pro. Google earth is freely available and accurate for land use land cover analysis which represents a powerful and attractive source of positional data that can be used for investigation and preliminary studies with suitable accuracy and low cost (Abineh Tilahun and Bogale Teferie.2015). The year 1990 and 2000 images of training data were collected from ground point supplemented by the elder local people. The main land uses identified in the watershed were cultivated land, dense forest, grazing land, open forest and settlement/homestead. These land uses are described in table 2.

Table3.2. LULC classes’ description (adopted from Gebremedhin Tesfaye et al., 2017)

| Land use /land cover type | Description |
| --- | --- |
| Cultivated land | This unit includes areas used for rained and irrigated cultivation, including nearly fallowed areas |
| Grazing land | Areas with permanent grass cover, use for grazing usually communal |
| Homestead | Areas with the settlement, garden around the home and organizational office |
| Open forest /woodland | Areas covered with scrap scattered trees and sparsely planted woody plant mixed with short bush open areas |
| Forest /dense | Areas covered with trees forming a closed canopy or relatively closed canopy. This unit also includes dense bush land mixed with closed wood trees |

## 3.4. Accuracy Assessment and Change Detection

Accuracy assessment is an important part of any classified image to another data source that is considered to be accurate or ground truth data. It is also the mandatory evaluation step during the image classification process, as the user of land cover output needs to determine how accurate the result is used the procedure data correctly (Manandhar et al., 2009).

A confusion matrix (or error matrix) were developed, expressing the number of sample units assigned to a particular category relative to the actual category, as verified on the ground (Lu & Weng, 2007). The confusion matrix encompases overall accuracy (OA), producer accuracy (PA), user accuracy (UA) and kappa coefficient (Thomlinson et al.1999). In land use and cover classification system, kappa values of more than 0.80 indicate a good classification performance. Kappa values between 0.40 and 0.80 indicate moderate classification performance and Kappa values of less than 0.40 indicate poor classification performance (Jensen, 2005; Lillesand et al., 2004).

Therefore, the classification accuracy can be defined as the number of pixels that is correctly classified to the sum of all pixels. It is predicted by evaluating overall accuracy along with the kappa coefficient. These values are calculated by using the confusion matrix (Lillesand et al., 2004). The kappa coefficient can be estimated based on the following mathematical equations.

Overall accuracy = total number of correctly classified pixels (diagonal) X 100 --- (2)                                 Total number of reference pixels

User accuracy =number of correctly classified pixels in each category X100 ----- (3)                             Total number of classified pixels in that category (the row total)

Producer accuracy = number of correctly classified pixels in each category X100 - (4)

Number of reference pixels in that category X 100

Kappa coefficient (t) = (TS*TCS)- ∑(Col.totXRow.tot)X100 ------------------------ (5)    TS2 - ∑(Col.totXRow.tot)

NB. TS= total no. of sample

TCS = total no. of corrected sample

Col.tot = column total

Row.tot = row total

In order to capture qualitative and quantitative information that helps to explain and interpret the land use and land cover change, a field survey was conducted. In this case, the historical land use and land cover trend have been collected through supplemented by the elder local people. Therefore, for each class, 75 GPS for land use land cover classification and 200 GPS for accuracy assessment were collected based on a random sampling technique. Since the 1990 and 2000 ground truth samples were collected supplemented by asking the elder local people with supporting GPS tools and the 2010 and 2020 periods were also collected from Google Earth historical imagery with supporting GPS tools.

## 3.5. Invest SDR Model Data Inputs

Various factors including LULC, soil, topography, climate, and management practices affect the rate of soil erosion and sediment yield in a watershed. In this study, multiple data including spatial and non spatial data were utilized. The data used in the study include watershed boundary shapefile (vector), LULC map (raster), rainfall erosivity (R-factor) raster, soil erodibility (K-factor) raster, DEM (raster), biophysical table (CSV C-factor and P-factor), threshold flow accumulation, Borselli K parameters, Borselli ICo parameters and Maximum SDR value,. All the raster and vector data were projected to UTM, datum WGS84 zone 37 N.

### 3.5.1. Watershed Boundary

A shapefile of the Maybar watershed was one of the input data to InVEST model. It was extracted from DEM using ArcGIS 10.3.1 and used to determine the boundaries of the watershed

### 3.5.2. LULC Data

The InVEST sediment delivery ratio model requires a raster LULC dataset with an integer LULC code for each cell. The LULC dataset was extracted from the Landsat image of 1990, 2000, 2010 and 2020 years which was downloaded from the USGS website (https://earth.explorer.usgs.gov). The LULC dataset was then created by employing supervised image classification using the maximum likelihood algorithm.

### 3.5.3. Rainfall erosivity factor (R)

The erosivity factor was calculated by using (Hurni, 1985), and derived from spatial regression analysis for Ethiopian highlands, based on the available mean annual rainfall (P). Fifteen year data of monthly and annual rainfall were obtained from four meteorological stations in and around the watershed (Dessie, Kombolcha, Albuko and Maybar). Therefore, the mean annual rainfall data at each station was used the Inverse Distance Weighted (IDW) interpolation technique in Arc GIS 10.3.1 to create spatial raster map. The IDW geostatistical interpolation method was preferred because it makes easier to generate relatively accurate rainfall data from known sample points located at closer distances than those located far from the points of unknown values. The method was also selected for the reason that it enables better interpolation of the required (Li and Heap, 2008). The interpolated R factor was used as input for the model.

R= (0.562*P) - 8.12 -------------------------------------------------------------------- (6)

Where R = the factor of erosivity and P = the mean annual precipitation in mm/year

Table3.3. The mean annual rainfall and corresponding R-factor

| Satiation | X | Y | Mean annual rainfall 2006-2020 year | R-factor |
| --- | --- | --- | --- | --- |
|  | (Easting) | (Northing) |  |  |
| Dessie | 569203 | 1232529 | 1333 | 741 |
| Kombolcha | 578375 | 1225307 | 1207 | 670 |
| Albuko | 570558 | 1202565 | 1503.8 | 837 |
| Maybar | 571891 | 1215702 | 1448 | 806 |

Figure 3. 2. Location of rain gauge stations in and around the study watershed

.

Figure 3 .3. The R-factor maps of the study watershed

### 3.5.4.. Soil Erodibility Factor (K)

One of the requirements for InVEST model is soil erodibility data. Soil erodibility is the soil susceptibility to erosion (Renard et al., 1997; Farhan and Nawaiseh 2015; Panagos et al., 2015). The K-factor reveals the ease with which the soil is removed by splash and surface flow. It also indicates the effect of soil properties on soil loss and the susceptibility of soil to erosion. Literature reveals that various approaches have been used by scholars to determine erodibility of soil depending on data availability (Hurni 1985; Römkens et al., 1997). For instance, a study by (Hurni, 1985) indicated that the K-factor can be determined depending on soil texture, organic matter content, permeability, grain size distribution, and other factors. In the present study, due to the paucity of data, a soil map was obtained from the geology is Volcanic Trapp series with alkali-olivine basalts Ethiopian map agency ( (SCRP, 2000) and K-values were assigned based on the recommendation of Hurni (1985) The soil of the catchment is mainly dominated by Eutric Leptosols which accounts 71.4% of the whole watershed while the remaining is Eutric Regosol 19.1% and Eutric Vertisols 9.5% (Table 4).

Figure3. 4. Soil types (a) and K-factor (b) maps of the study watershed

Table3.4. The soil types and the corresponding K-values recommended by Hurni (1985)

| Soil type | Area |  | K-value | Reference |
| --- | --- | --- | --- | --- |
|  | ha | % |  |  |
| Eutric Leptosols | 83 | 71.4 | 0.2 | Hurni (1985) |
| Eutric Regosol | 22.17 | 19.1 | 0.2 |  |
| Eutric Vertisols | 11.1 | 9.5 | 0.15 |  |
| Total | 116.27 | 100 |  |  |

### 3.5.5. Topographic (LS) Factor

The 30 m spatial resolutions ASTER DEM (Aster digital elevation model) were used to generate slope. The flow accumulation and slope steepness was computed from the DEM using Arc GIS 10.3.1. Flow Accumulation was derived after conducting Fill and Flow Direction processes by using Arc Hydro tools, in the GIS software. Flow accumulation and slope maps were multiplied Raster Calculator in Arc GIS 10.3.1 to calculate and map the slope length (LS factor) as suggested by Moore and Burch (1986 a & b), and employed by Abate Shiferaw (2011a), Prasannakumar et al. (2012), Nigussie Haregeweyn et al. (2017), Ostovari et al. (2017) and Mengesha Zerihun et al. (2018).

LS = (Flow Accumulation*Cell size / 22.13)0.4*(Sin slope / 0.0896)1.3 ------------ (7)

Where, cell size is the resolution of the ASTER GDEM (here 30 m), S is the slope gradient (in degree)

Figure 3.5. The slope (in degree) (a) and LS-factor (b) maps of the study watershed

### 3.5.6. Biophysical CSV Table

For the calculation of erosion and sediment yield a CSV table containing data on cover management C_factors and support practice P_ factors corresponding to each of the LULC classes was required. In the table, rows were LULC classes and columns were named as lucode, RUSLE_C or P in which they represent land use code, land cover and management factor, and support practice factor respectively. The lucode was land use code of a unique integer for each LULC class which was matched to the LULC raster input.

### 3.5.7. Land Use/Land Cover Management Factor (C-Factor)

The RUSLE_C values indicate how the covers of the land types (such as cultivated land, dense forestry, etc.) affect soil loss (Renard et al. 1997; Nigussie Haregeweyn et al., 2017). Determining RUSLE_C values entails data related to soil management conditions, the nature of plant canopy and crop residues as a soil cover, soil surface roughness and the level of soil moisture. However, estimating each of these parameters was difficult due to the paucity of data (Renard et al., 1997; Farhan and Nawaiseh, 2015). In most cases LULC map and Normalized Diference Vegetation Index (NDVI) are used for RUSLE_C value estimation (Karaburun 2010; Lin et al., 2017). In this study the LULC map approach was selected since it gives a comparatively precise RUSLE_C value than the NDVI (Lin et al., 2017). To assign RUSLE_C values for each LULC class, the raster data was converted to vector format using Arc GIS10.3.1 the RUSLE_C values were assigned based on literature suggestions for the highlands of Ethiopia. (Benavidez et al., 2018) recommends that a simpler method of determining the RUSLE_C values is using values from studies that have reported values for similar LULC or from studies conducted in the same local area or region. However the definition of LULC type differs between studies. Identifying the variation between LULC classifications before applying RUSLE_C values from the studies was important. Hence to reduce uncertainty in the estimated values of RUSLE_C values from those studies that have similar LULC classifications were adopted. Finally, the adopted values were found to be floating point values between 0 and 1.

Table3.5. Adopted values of RUSLE C_ factor for different LULC classes

| No | LULC type | RUSLE_C values | Reference |
| --- | --- | --- | --- |
| 1 | Cultivated land | 0.5 | (Yang et al., 2003; Theobald et al., 2010) |
| 2 | Dense forest | 0.001 | Hurni (1985), Reusing et al. (2000), Morgan (2005), Mengesha Zerihun et al. (2018) |
| 3 | Grazing land | 0.05 | (Hurni 1985; Woldeamlak Bewket and Ermias Teferi 2009) |
| 4 | Open forest | 0.014 | Wischmeier and Smith (1978); Abate Shiferaw (2011a); Habtamu Sewnet and Amare Sewnet (2016); Desalew Meseret and Bhat (2017); Temesgen Gashaw et al. (2017a) |
| 5 | settlement | 0.003 | Kayet et al. (2018) |

Figure 3. 6. The C-factor maps of the study watershed in 1990 (a), 2000 (b), 2010 (c) and 2020(d) years.

### 3.5.8. Support Practice Factor (P-Factor)

The P-factor represents the role of conservation practices in reducing erosion (Wischmeie et al., 1978). The value of P-factor is between 0 and 1. In general, 1 is assigned to areas without protection measures (Temesgen Gashaw et al., 2019, Adornado et al., 2009), and a minimum value close to 0 is given for areas with suitable protection measures. Therefore, the lower the P value is the more effective the protection against erosion is (Prasannakumar et al., 2012). A review of the RUSLE model by Benavidez et al., (2018) emphasized that the P-factor could also be estimated using sub factors. The difficulty of accurately mapping supporting practice factors or not observing support practices has led many studies to ignore it by setting the value of its P-factor to 1, as seen in other studies (Marondedze, A.K. and Schütt, B., 2020, Temesgen Gashaw et al., 2019, and Adornado et al., 2009). However In this study, there was only a small area that has been left untreated with conservation structures through the agricultural extension campaign of the government. However, the conservation structures are poorly implemented due to their top-down approach. Considering the lack of data in the area on permanent management factors, the P values, suggested by Hurni (1985), Wischmeier and Smith (1978) and Sharma et al. (2011) that considers only two types of land uses (agricultural and nonagricultural) and land slopes, were used in this study (Table 3.6). In this study, the classifcation statistics in ArcGIS calculated the mean P value of 0.5.

Table3.6. Adopted values of RUSLE P-factor for different LULC classes

| Land use | Slope Catagories | P-factor values | References |
| --- | --- | --- | --- |
| Cultivated lands | 0-5 | 0.10 | Wischmeier and Smith (1978) and Hurni (1985) |
|  | 5-10 | 0.12 |  |
|  | 10-20 | 0.14 |  |
|  | 20-30 | 0.19 |  |
|  | 30-50 | 0.25 |  |
|  | 50-100 | 0.33 |  |
| Other land uses | All | 1 |  |

Figure 3 .7. The P-factor maps of the study watershed

## 3.6. Model Structure

InVEST sediment yield model helps the mapping and quantification of annual soil erosion and sediment export in a watershed. It calculates soil erosion and sediment delivery in a spatially explicit manner working at the spatial resolution of the input DEM raster. For each cell of the output data, the model primarily calculates the amount of eroded soil and then computes the sediment delivery ratio (SDR)  which is the amount of soil loss that reaches a watershed’s outlet (Sharp et al., 2018). The approach was developed by Borselli et al., (2008) and has received increasing attention in recent years (Cavalli et al., 2013; López-vicente et al., 2013)

## 3.7. Annual Soil Erosion

To determine the amount of annual soil loss on pixel (t/ha/year), the model uses the RUSLE (Sharp et al., 2018). The equation estimates water caused soil loss for varying climatic, soil, and topographic conditions. Since its development, RUSLE has been continuously improved to more precisely calculate soil loss and to adapt to a varying range of geographic areas. The equation is widely applied and is explained by the following equation (Renard et al., 1997).

RUSLE = R∗K∗LS∗C∗P --------------------------------------------------------------- (8)

Where RUSLE is the mean annual soil loss in t /ha/year, R is the rainfall erosivity in millimeter per hectare per hour per year (mm/ ha/year). which is derived from daily precipitation data; K is the soil erodibility factor in (t/ ha/ hour/year) which is derived from data on soil types; LS is the slope length-gradient factor which is the length of the slope and percent of the slope steepness derived from DEM (dimensionless); C is the land cover and management factor (dimensionless) which is derived from LULC classification of satellite image data; and P is the support practice factor which accounts for soil erosion control measures (dimensionless) derived from LULC classification of satellite image data.

## 3.8. Annual Sediment Export

The sediment export is the proportion of soil loss reaching the nearby streams (Sharp et al., 2018). The model estimates the exported sediment based on the work by (Borselli et al., 2008). Since the estimation of SDR at each pixel is determined by the upslope area and down slope flow path, the model first computes the connectivity index (IC) which is given by the equation no 9

IC = log10 (DupDdn) ------------------------------------------------------------------------------ (9)

Dup is the upslope component and given by

Dup = CS  Ax ------------------------------------------------------------------------------- (10)

Where C is the mean C factor of the upslope contributing area; S is the mean slope gradient of the upslope contributing area; and A is the upslope contributing area in m2 which the model delineates based on the D-infinity flow algorithm (Tarboton 1997; Sharp et al., 2018). The down slope component (Ddn) is defined as:

Ddn=i=1ndicisi     --------------------------------------------------------------- (11)

where di is the length (in m) of the flow path along the ith cell based on the steepest downslope direction; Ci and Si represent the C factor and the slope gradient of the ith cell, respectively. The model determines the downslope fow path using the D-infnity fow algorithm (Tarboton, 1997; Sharp et al., 2018).

Then, the model computes the SDR ratio for a pixel i from the connectivity index (IC) based on (Vigiak et al., 2012).

SDRi=SDRmax1+expICO-ICik  --------------------------------------------------------------- (12)

Where SDRmax is the maximum hypothetical SDR, set to an average value of 0.8 (Vigiak et al., 2012), and IC0 and k are calibration values that determine the shape of the SDR-IC relationship (increasing function) (Sharp et al., 2018).

The sediment load from a given pixel i, Ei (t/ha/year) is given by:

Ei=uslei*SDRi  -------------------------------------------------------------------------- (13)

The total sediment load from the watershed, E (t/ha/yr) is given by:

E=i=1nEi   --------------------------------------------------------------------------------- (14)

## 3.9. Methods of Data Analysis

The data analysis was done by using GIS software, InVEST SDR model, and Excel Before analysis, data coding of responses and analysis were make. To analyze the data obtained easily, the data were coded to GIS software and InVEST SDR software. This task involved identifying, classifying, and assigning a numeric or character symbol to data. The analysis included exploring the relationship between variables and comparing groups how they affect each other.

To analysis land use land cover change the watershed area were categorize into different LULC classes by using the supervised image classification techniques in GIS software. A confusion matrix (error matrix) was used to measure the reliability of the LULC classification and validation by using GIS software and excel by intersecting different year maps..

To analysis or computed soil erosion and sediment yield combine the datasets and run the model, all the input data were set to the same spatial resolution, projection, and reference system. After preparing and arranging the input data all the parameters were combined using InVEST 3.8.9 model to generate final estimated values of soil erosion and sediment export.

The outputs from the InVEST sediment yield model included the amount of soil erosion, sediment eroded in the catchment, the sediment retained by the vegetation and topographic features as well as the sediment load beyond the retention capacity of the vegetation and topographic features which are delivered to a water body at an annual time scale. These outputs are important to estimate the regulatory capacity of the watershed’s LULC for soil erosion and sediment protection services, which are vital in studying the management of the lake, reservoir, and water quality in streams (Sharp et al., 2018). Finally, the spatial distribution of the estimated mean annual soil erosion and sediment export were presented using maps and tables.

## 3.10. Data Validation

To test the applicability and reliability of the models, a comparison of simulated results against the observed data is necessary. Validations of the SDR model of the InVEST were undertaken by comparing the annual simulate data against observe data. These observe data (location of these gauging stations, it was obtained from Sirinka Agricultural Research Center. Model performance was evaluated using statistics recommended by Moriasi et al. (2007); Boskidis et al. (2012). The performance of the model related to sediment yield was assessed by Percent bias (PBIAS) (Moriasi et al. 2007), coefficient of determination (R2) (Legates and McCabe 1999) and RMSE-observations standard deviation ratio (RSR) (Sorooshian et al. 1993) and their performance rating statistics are presented in Table 7. The capabilities of model performance measures of R2 and RSR were reported by (Abbaspour et al. 2007; Setegn et al. 2010; Arnold et al. 2012; Fiseha et al. 2014, 2012).

R2: as a standard regression, it describes the degree of co linearity between simulated and measured sediment yield. The range of R2 lies between 0 and 1

R2=i=1n (oi-oi min) (si-simin)i=1n (oi-oi min)2  i=1n (si-si min)2  2  --------------------------------- (15)

Where oi is the observed value and si is the simulated value oi min is the mean observed and si min is the mean simulated value of the model for the entire evaluation time period, and n is the total number of observations.

PBIAS: Gupta et al. (1999) described PBIAS as the measure of the average trend of simulated data either to be smaller or larger from the observed counterparts. PBIAS is calculated as shown in Eq. (21). The optimal value of PBIAS is 0 and lower values represent correct model simulation. Positive and negative values indicate model underestimation and overestimation bias, respectively (Gupta et al. 1999).

PBIAS= i=1n (Oi-Si)i=1n Oi*100  --------------------------------------------------------- (16)

RSR is the ratio of the root mean square error (RMSE) and the standard deviation of measured data (Moriasi et al. 2007) as shown in Eq. (22). RSR ranges between its optimum values of 0, which indicates model perfect simulation, to positive values.

RSR=RMSESTDEobsi=1n (oi-si )2)i=1n (oi-oi min)2  2   ----------------------------------------- (17)

Table3.7. Objective functions and model performance ratings for simulations of sediment yield.

| Objective Function | value range | Performance Classification | References |
| --- | --- | --- | --- |
| R2 | 0.7 < R2 < 1 | Very good | Moriasi et al. (2007) |
|  | 0.6 < R2 < 0.7 | Good |  |
|  | 0.5 < R2 < 0.6 | Satisfactory |  |
|  | R2 < 0.5 | Unsatisfactory |  |
| RSR | 0.00 ≤ RSR ≤ 0.50 | Very good | Moriasi et al. (2007) Boskidis et al. (2012) |
|  | 0.50 < RSR ≤ 0.60 | Good |  |
|  | 0.60 < RSR ≤ 0.70 | Satisfactory |  |
|  | RSR > 0.70 | Unsatisfactory |  |
| PBIAS | PBIAS < ±15 | Very good | Moriasi et al. (2007) |
|  | ±15 ≤ PBIAS < ±30 | Good |  |
|  | ±30 ≤ PBIAS < ±55 | Satisfactory |  |
|  | PBIAS ≥±55 | Unsatisfactory |  |

# 4. REULTS AND DISCUSSIONS

## 4.1. LULC Changes in the Study Watershed from 1990 to 2020

The area coverage and proportion of LULC change for the years 1990, 2000, 2010, and 2020 are presented in Tables4.1 and Figures 4.1and 4.2. The results indicated that land use land cover of the area was changed either from one land use type to the other following increasing or decreasing pattern. Both the dense forest and homestead area were increased from 19.8% to 39.1% and 12.9% to 20.7%, from 1990 to 2020 years respectively. This change may be explained by different factors. The drivers could be changes in agricultural practices which themselves are influenced by institutional and economic factors, public policy and infrastructure growth (Counties et al., 2014). In the study area it has been observed that the involvement of different governmental and nongovernmental organizations in the integrated watershed development was high. Moreover improvement of infrastructure (electricity, road, health center and school), plantation forest, and house building were increased.

Watershed interventions could increase the vegetative index or greenery and income thus reduce poverty. More over improvement of infrastructure such as electricity, road, health center and school may be reduction of migration from the watershed to other areas mostly to urban areas (Pathak et al., 2013). Therefore reduction of migration and improving livelihood strategy is the means of increasing the settlement area. The result is in line with In most parts, old agricultural lands that exhausted crop production potential are abandoned to area enclosure as well as tree planting is the means of increase dense forest and decrease the cultivated land (Gete .Zeleke, 2000, Woldeamlak Bewket, 2003, Amare. Bantider, 2007, Githui et al., 2009, Abate Shiferaw, and Singh, K.L., 2010, Ermias Teferi et al., 2010, Wagner et al., 2013, Abonesh, Tesfaye et al., 2014, Wagesho, 2014). Our finding also agreed with this reports.

The result is also consistence with the findings of the aforementioned studies in different parts of the country, (Amare Bantider et al., 2011) reported the expansion of forest area and shrub land and the decline of the area of cropland since the mid-1980s in the Eastern Escarpment of Wollo, Northeastern Ethiopia. (Asmamaw, L.B et al., 2011) have also shown the expansion of forest land during the study period between 1958 and 2006 in Gerado Catchment of Northeastern Ethiopia.

In contrast, the result shows that that cultivated land were gradually decreased from 25.51% in 1990% to 20.8% in 2010 and somewhat increase in 2020 to 22%., open forest and grazing land is decreased 31.47% to 16.5% and 10.32% to 1.7%, from 1990 to 2020 study years respectively

During the field visit, at the time of land use classification it has been observed that most previous cultivation land, grass land and open forest land were changed to a plantation dense forest and settlement land. This might be due to the benefits of forest is more than other in the area for its multiple uses including cash income. Similarly, the study also conducted on landuse land cover that resulted in a slight increase in the proportion of dense forestland, while cultivated land, grass land and open forests have decreased (Dessie, Assefa et al., 2017; Gebremedhin, Tesfaye et al., 2017; Mohajane, M., 2018). This investigation has supported most of the integrated watershed management and land use land cover change studies. In Kenya, a study in land use land cover changes over a period of 30 years reported that agricultural land reduced from 39.7% to 15.8% (Musa, M.K, 2015). The result of the present research is generally consistent with the findings of other studies in the highlands of Ethiopia.

Figure 4. 1. Land use land cover classification map of 1990; 2000; 2010 and 2020 years

Table4.1. Land use land cover classification of 1990; 2000; 2010 and 2020 years

| Lulc change | 1990 |  | 2000 |  | 2010 |  | 2020 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Area (ha) | % | Area (ha) | % | Area (ha) | % | Area (ha) | % |
| Cultivated land | 29.67 | 25.51 | 27.2 | 23.4 | 24.22 | 20.8 | 25.63 | 22 |
| Dense forest | 23 | 19.8 | 29.31 | 25.2 | 42.65 | 36.68 | 45.44 | 39.1 |
| Grazing laand | 12 | 10.32 | 6.4 | 5.5 | 3 | 2.58 | 2 | 1.7 |
| Open forest | 36.6 | 31.47 | 32 | 27.5 | 24 | 20.64 | 19.2 | 16.5 |
| Home stead | 15 | 12.9 | 21.36 | 18.4 | 22.4 | 19.3 | 24 | 20.7 |
| Total | 116.27 | 100 | 116.27 | 100 | 116.27 | 100 | 116.27 | 100 |

Figure 4. 2. Land use land cover

### 4.1.1. Accuracy of the Classified Images

The accuracy report of the four classified images is shown in table 4.2. The diagonals of the matrix are the correctly classified values while the off diagonals are the misclassified values. The overall accuracy of the classification is 87.5, 88, 90% and 90.5% for the period 1990, 2000, 2010 and 2020 respectively. A Kappa coefficient of above 0.80 was also obtained for the four classified images. According to Monserud (1990), a Kappa value between 0.70 and 0.85 are generally rated as very good. Therefore, in this study the validation data set indicated a very good agreement of the classified image with the ground truths.

Table4.2. Accuracy assessment in percent % of lulc change map of each year

| Lulc class | 1990 |  | 2000 |  | 2010 |  | 2020 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | user | Producer | user | producer | user | producer | user | producer |
| Cultivation | 91.1 | 94.8 | 87.6 | 95 | 94.33 | 89.28 | 94 | 85.45 |
| Dense forest | 80 | 91.4 | 90 | 90 | 88.33 | 94.64 | 84.37 | 98.1 |
| Grass land | 90 | 78 | 84 | 77.7 | 86.6 | 76.4 | 100 | 100 |
| Open forest | 90.7 | 86.7 | 90 | 85.7 | 94.2 | 89.09 | 94.1 | 61.5 |
| Home stead | 73.3 | 68.7 | 80 | 80 | 75 | 93.7 | 89.31 | 100 |
| Over all | 87.5 |  | 88% |  | 90% |  | 90.5 |  |
| Kappa | 83.5 |  | 83.97% |  | 86.77% |  | 87.68 |  |

Accuracy assessment was performed for the years 1990, 2000, 2010, and 2020, based on Land-use land cover change maps, Google earth pro at different times of the year, and knowledge of the local community. The classified results of 80% are acceptable; this accuracy is within specified as minimum accuracy standards by the USGS. The results from accuracy assessment showed an overall accuracy in acceptable ranges which are 87.5%, 88%, 90%, and 90.5% with corresponding kappa statistics of 83.5%, 83.97%, 86.77%, and 87.68% for the year 1990, 2000, 2010, and 2020 respectively (Table 4.2) and the overall accuracy results of these study was accurate which is a satisfactory accuracy standard for such assessment (Varamesh, S., Hosseini, S.M. and Rahimzadegan, M., 2017).

## 4.2. Land Use Land Cover Change Detection

The changes in land use land cover in hectar and percentage over a three decadal period (Table 4.3 and figure 4.3) indicated that dense forest and homestead positively changed. While grassland and open forest showed negative change throughout the whole study period. On the other hand, some fluctuation on the cultivation land in 2010–2020 land use cover classes has increased however, its general trend shows negatively changed.

According to Amare Sewnet, (2015), in the northwestern Ethiopia, in treated watershed bush lands were lost and occupied by cultivated lands and forest covers and due to the population growth, the settlement area also increased. Contrary to this there are reports of decreased forest area in untreated watersheds. A case in point is a study in Wallecha Watershed in the Bilate River Basin in Wolayita Zone; the general trend was a decrease in forest lands and shrub grasslands while cultivated lands were increasing (Babiso, B., Toma, S. and Bajigo, A., 2016).

Table4.3. Land use and land-cover change detection

| Lulc change | 1990-2000 |  | 2000-2010 |  | 2010-2020 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Area (ha) | % | Area (ha) | % | Area (ha) | % |
| Cultivated land | -2.47 | -8.32 | -2.98 | -10.9 | 1.41 | 5.8 |
| Dense forest | 6.31 | 27.4 | 13.34 | 45.5 | 2.79 | 6.5 |
| Grazing land | -5.6 | -46.6 | -3.4 | -53.1 | -1 | -33.3 |
| Open forest | -4.6 | -12.56 | -8 | -25 | -4.8 | -20 |
| settlement | 6.35 | 42.4 | 1.04 | 4.86 | 1.6 | 7.1 |

Figure 4.3. Land use land cover detection

## 4.3. Analyses of LULC Conversions

The conversions taken place to and from one LULC category to another during 1990 to 2000, 2000 to 2010 and 2010 to 2020 periods are presented in Table 4.4, 4.5 and 4.6. The diagonals of the matrix from the tables are the persistence while the off diagonals are the conversions from one category to the others. The change detection analyses indicated the significant conversions in LULC in both periods.

### 4.3.1. Conversions between 1990 and 2000

In these periods, 8.9 ha, 0.8ha, 5.97 ha, 1.7ha and 2.76ha, 3.64ha, 7ha, 1.5ha of cultivated land were converted to and converted from  dense forest, grassland, open forestland and settlement area, respectively. During these times, 2.76ha, 0.46ha, 2.55ha, 0.03ha and 8.9, 1.56ha, 1.5ha, 0.15ha, of dense forest land was also converted to and converted from cultivated land, grassland, open forestland and settlement area respectively. Although some proportion 3.64ha, 1.56ha, 3ha, 1.4ha and 0.8ha, 0.46ha, 2.7ha, 0.04ha of the grazing land area were also converted to and converted from cultivated land, dense forest land, open forestland and settlement area, respectively. On the other hand 7ha, 1.5ha, 2.7ha, 9.4ha and 5.97ha, 2.55ha, 3ha, 4.48ha of open forest land was converted to and converted from cultivated land, dense forest land, grazing land and settlement area respectively. In this year 1.5ha, 0.15ha, 0.04ha, 4.48ha and 1.7ha, 0.03ha, 1.4ha, 9.4ha of settlement area was also converted to and converted from cultivated land, dense forest area, grazing land and open forest areas respectively.

Table4.4. Transition area matrix in hectare between 1990 to 2000 periods in the maybar watershed

| 1990 | 2000 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | cultivated | dense forest | grazing land | open forest | settlement | Total |
| Cultivated | 12.3 | 8.9 | 0.8 | 5.97 | 1.7 | 29.67 |
| dense forest | 2.76 | 17.2 | 0.46 | 2.55 | 0.03 | 23 |
| grazing land | 3.64 | 1.56 | 2.4 | 3 | 1.4 | 12 |
| open forest | 7 | 1.5 | 2.7 | 16 | 9.4 | 36.6 |
| Settlement | 1.5 | 0.15 | 0.04 | 4.48 | 8.83 | 15 |
| Total | 27.2 | 29.31 | 6.4 | 32 | 21.36 | 116.27 |

### 4.3.2. Conversions between 2000 and 2010

In these periods, 9.38 ha, 0.04ha, 3.77 ha, 2.51ha and 3.15ha, 0.5ha, 5.6ha, 3.47ha of cultivated land were converted to and converted from dense forest, grassland, open forestland and settlement area, respectively. During these times, 3.15ha, 0.11ha, 1ha, 0.1ha and 9.38ha, 1.06ha, 5.9ha, 1.36ha, of dense forest land was also converted to and converted from cultivated land, grassland, open forestland and settlement area respectively. Although some proportion 0.5ha, 1.06ha, 1.7ha, 1.4ha and 0.04ha, 0.11ha, 0.57ha, 0.14ha of the grazing land area were also converted to and converted from cultivated land, dense forest land, open forestland and settlement area, respectively. On the other hand 5.6ha, 5.9ha, 0.57ha, 6.47ha and 3.77ha, 1ha, 1.7ha, 4.06ha of open forest land was converted to and converted from cultivated land, dense forest land, grazing land and settlement area respectively. In this year 3.47ha, 1.36ha, 0.14ha, 4.06ha and 2.51ha, 0.1ha, 1ha, 6.46ha of settlement area was also converted to and converted from cultivated land, dense forest area, grazing land and open forest areas respectively.

Table4.5. Transition area matrix in hectare between 2000 to 2010 periods in the maybar watershed

| 2000 | 2010 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | cultivated | dense forest | grazing land | open forest | Settlement | Total |
| Cultivated | 11.5 | 9.38 | 0.04 | 3.77 | 2.51 | 27.2 |
| dense forest | 3.15 | 24.95 | 0.11 | 1 | 0.1 | 29.31 |
| grazing land | 0.5 | 1.06 | 2.14 | 1.7 | 1 | 6.4 |
| open forest | 5.6 | 5.9 | 0.57 | 13.47 | 6.46 | 32 |
| settlement | 3.47 | 1.36 | 0.14 | 4.06 | 12.33 | 21.36 |
| Total | 24.22 | 42.65 | 3 | 24 | 22.4 | 116.27 |

### 4.3.3. Conversions between 2010 and 2020

In these periods, 3.72 ha, 0.1ha, 2.4 ha, 8ha and 8ha, 0.3ha, 4.83ha, 2.5ha of cultivated land were converted to and converted from  dense forest, grassland, open forestland and settlement area, respectively. During these times, 8ha, 0.07ha, 3ha, 4.58ha and 3.72ha, 0.44ha, 5.98ha, 8.3ha, of dense forest land was also converted to and converted from cultivated land, grassland, open forestland and settlement area respectively. Although some proportion 0.3ha, 0.44ha, 0.8ha, 0.12ha and 0.1ha, 0.07ha, 0.29ha, 0.2ha of the grazing land area were also converted to and converted from cultivated land, dense forest land, open forestland and settlement area, respectively. On the other hand 4.83ha, ha, 5.98ha, 0.29ha, 4.9ha and 2.4ha, 3ha, 0.8ha, 5ha of open forest land was converted to and converted from cultivated land, dense forest land, grazing land and settlement area respectively. In this year 2.5ha, 8.3ha, 0.2ha, 5ha and 8ha, 4.58ha, 0.12ha, 4.9ha of settlement area was also converted to and converted from cultivated land, dense forest area, grazing land and open forest areas respectively.

Table4.6. Transition area matrix in hectare between 2010 to 2020 periods in the maybar watershed

| 2010 | 2020 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Cultivated | dense forest | grazing land | open forest | settlement | Total |
| Cultivated | 10 | 3.72 | 0.1 | 2.4 | 8 | 24.22 |
| dense forest | 8 | 27 | 0.07 | 3 | 4.58 | 42.65 |
| grazing land | 0.3 | 0.44 | 1.34 | 0.8 | 0.12 | 3 |
| open forest | 4.83 | 5.98 | 0.29 | 8 | 4.9 | 24 |
| Settlement | 2.5 | 8.3 | 0.2 | 5 | 6.4 | 22.4 |
| Total | 25.63 | 45.44 | 2 | 19.2 | 24 | 116.27 |

## 4.4. Soil erosion and sediment export in the maybar watershed

### 4.4.1. Soil erosion

Understanding the level of soil erosion and sediment export are important for science based sustainable management of natural resources in a watershed. The study found that the mean soil loss from the watershed was ranges from 21.06-28.86 t/ ha/year which is 28.86, 25.12, 21.06 and 26.16 t/ha/year, in the study period 1990, 2000, 2010 and 2020 years respectively. The soil loss is decreasing during the study period 1990, 2000 and 2010 because it may be due to the implementing of extensive soil and water conservation measures through community lead watershed management and initiatives such as Productive Safety Net Programs (PSNP), community mobilization through free labor days, and the Sustainable Land Management Project (SLM). Mainly the implementation includes terracing, soil and stone bund, and biological conservation measures. These interventions are further supported by the agricultural extension system and community participation. This mass movement of Soil Water Conservation (SWC) highlights significant progress in land management practices in the study area. However, to achieve sustainable ecosystem services conservation, continuing maintenance of structures and the consistent application of a scientific methodology are necessary. The other reason may be due to shift from baseline to intensive cultivation land management, and using these areas for area enclosure and planting tree. But in 2020 soil loss indicates increased this may be due to farmers participation on integrated watershed management is decrease, the government attention is focused on other political situation, increasing of cultivated land and previous work is destroyed.

The result  is consistency with the previous report Geleda watershed Blue Nile Basin, Ethiopia 23.7 t/ha/year RUSLE (Temesgen Gashaw et al.,  2017a), Upper Blue Nile basin 27.5 t/ha/year RUSLE (Nigussie Haregeweyn et al., 2017), Jabi Tehinan watershed 30.4 t/ha/year RUSLE (Tadesse Amsalu and Abebe Mengaw, 2014), eastern escarpment of Wondo Genet watershed 26 t/ha/year RUSLE (Amare Sisay et al., 2014), Borena Woreda of South Wollo highlands, 27 t/ha/year RUSLE (Abate Shiferaw, 2011) and Ethiopian highlands 35 t/ha/year USLE (FAO, 1986) and (Nigussie Haregeweyn et al., (2015)  who reported the average soil erosion rate in Ethiopia to be 29.9 t/ha/ year.

Overall, compared to the soil loss estimated values in this study, the figures in the literature have disparity and inconsistency. Such disparity may arise from the use of different methods and the variations in the biophysical environment and management practices. The estimated soil loss value is higher compared to the soil formation rates of different places in Ethiopia, which were estimated to be in a range of 2–22 t/ha/year (Hurni, 1983). Besides, the value is higher compared to the soil loss tolerance limit in Ethiopia, which was suggested by Hurni, (1986) to be in a range of 2–18 t/ha/year on agricultural lands, and which was estimated by Morgan (1995) to be 10 t/ha/ year . Moreover, the value is higher than the soil loss rate that can be reversed within a time span (Kouli et al., 2009) reported that an erosion rate which is above 10 t/ha/ year will not be reversed within a period of 50 to 100 years.

However, the computed erosion rate in the watershed is less than the highest rates of erosion in Ethiopia which were reported by Gete Zeleke, (2000) in northwestern highlands of Ethiopia 243 t/ha/year, (Mesfin Sahle et al., 2019) in the Wabe River catchment 165 t/ha/year, and (Nigussie Haregeweyn et al., 2015) in Anjeni 110 t/ha/year and Chemoga 102 t/ha/year watersheds in Ethiopia.

Table4.7. The total soil erosion and mean soil erosion rated of entire watershed during 1990-2020 periods.

| year | Mean annual soil erosion rate t/ha/year |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Cultivated land | Dense forest land | Grazing land | Open forest | Settlement land | Entire watershed |
| 1990 | 68 | 14 | 27 | 18 | 7 | 28.9 |
| 2000 | 44 | 14 | 25.5 | 29 | 12 | 25.12 |
| 2010 | 70 | 5.5 | 16 | 16.8 | 8 | 21.06 |
| 2020 | 86 | 3 | 24.5 | 17 | 18 | 26.16 |

Figure 4. 4. Soil loss map of each land use t/ha/year

In connection with LULC, the study indicated a considerable effect of the type of LULC on the extent of soil erosion in the maybar watershed. The highest contribution to the total soil loss in the watershed was from cultivated land. The reasons of higher soil loss in cultivated land may be due to poor farming practices, like burning of cow dung and crop residues (instead of leaving them on the field), poor soil management or use of no farm inputs, were identified as a major cause of high soil erosion in cultivated land.

The result of cultivated land mean soil loss is 68, 44, 70 and 86 t/ha/year in 1990, 2000, 2010 and 2020 year respectively which is in between with the soil erosion assessment in the northwestern highland, Chemoga watershed reported mean soil loss of cultivated land 93 t/ ha/year (Woldeamlak Bewket and Ermias Tefer, 2009), Similarly a study conducted in Koga watershed reported mean soil erosion rate of 47.4 t/ha/year (Habtamu, Sewnet Gelagay, 2016). In this regard, the result is comparable to studies which were conducted in other catchments which found cultivated has the highest contribution to a watershed’s soil loss (Sharma et al., 2011; Asnake Yimam and Amare Bantider 2019). Abreham Berta et al. (2020) reported mean soil loss of 80 t/ha/ year from cultivated fields in the Ethiopian highlands

Conversely, the lowest mean soil loss was obtained in dense forest land and settlement area which is 0.83 5, 4, 8 and 2, 3, 3, 5 t/ha/year in 1990, 2000, 2010 and 2020 periods respectively. Major effects of vegetation in reducing erosion are may be due interception of rainfall by absorbing the energy of the raindrops and thus reducing surface sealing and runoff, retardation of erosion by decreased surface velocity, physical restraint of soil movement, root regulating capacity of the vegetation, increased biological activity in the soil and transpiration, which decrease soil water, resulting in increased storage capacity and less runoff.

Table4.8. The contributions of eache LULC type to the total soil loss in the study watershed during 1990-2020 periods.

| year | soil loss (tones) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Cultivated land | Dense forest land | Grazing land | Open forest | Settlement land | total |
| 1990 | 2017 | 322 | 324 | 658.8 | 105 | 3426.8 |
| 2000 | 1196.8 | 410.3 | 163.2 | 928 | 256.3 | 2954.6 |
| 2010 | 1695.4 | 234.5 | 48 | 396 | 179 | 2575.9 |
| 2020 | 2204 | 136 | 49 | 326 | 432 | 3147 |
|  | Percent% |  |  |  |  |  |
| 1990 | 59 | 9.4 | 9.4 | 19.2 | 3 | 100 |
| 2000 | 40.5 | 13.9 | 5.5 | 31.4 | 8.7 | 100 |
| 2010 | 66 | 9 | 2 | 16 | 7 | 100 |
| 2020 | 70 | 4.1 | 1.5 | 10.4 | 14 | 100 |

### 4.4.2. Sediment export

Sediment export is the sediment beyond retention capacity of the vegetative cover and management practices that has the potential to reach the nearby streams. It is the sediment amount that can be compared to any observed sediment loading at the outlet of a watershed. For estimating the sediment export in each cell, the model first computes the amount of eroded sediment, then the sediment delivery ratio (SDR), which is the amount of soil loss that actually reaches the nearby streams and outlet of the watershed.

The result revealed that the computed total annual sediment export from the watershed was 226,690.3 tons. The mean annual sediment export from the watershed was in a range of 3.6 to 6.4 tons which is 6.4, 4.8, 3.6 and 5.07 t/ha/ year in 1990, 2000, 2010 and 2020 period respectively. The sediment export is decreasing during the study period 1990, 2000 and 2010 because it may be due to the implementing of different soil water conservation measures this finding was consistent with some similar studies, which also showed a decline in sediment yield due to land use change and soil conservation projects (Chu et al., 2009; Liu et al., 2013).

The result is consistence with previous report by Nyssen et al., (2007) northern Ethiopian highlands MayZegzeg catchment 5.6 t/ha/year sediment yield. It is also comparable to other sediment yield estimations in Ethiopia. For instance, (Nigussie Haregeweyn et al.,  2012) estimated a 2 to 19 t/ ha/year of sediment export from 14 micro dam watersheds in northern Ethiopia. (Sahle et al., 2019) also identified a sediment export of 0–33 t/ha/year from the Wabe River catchment of the Gibe basin. The estimated sediment export value was comparatively low to the reported figures. This is likely due to the remarkable difference in the climate, elevation, terrain, drainage density, soil types and conditions, geology, sizes of the watershed, and LULC between the study areas. However, field observations indicate that mass of soil materials that were removed from steep slopes of the watershed have been accumulated in downstream areas. Such exported sediment created problems of lake sedimentation, pollution of water bodies, and sediment deposition on the active agricultural lands.

Table4.9. The total sediment export and mean sediment export of entire watershed during 1990-2020 periods.

| year | Mean annual sediment yield t/ha/year |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Cultivated land | Dense forest land | Grazing land | Open forest | Settlement land | Entire watershed |
| 1990 | 15 | 3.5 | 6 | 3.4 | 1.5 | 6.4 |
| 2000 | 8.28 | 2.6 | 5.2 | 5.54 | 2.2 | 4.8 |
| 2010 | 11.2 | 1 | 2.3 | 3 | 1.15 | 3.6 |
| 2020 | 16.45 | 0.5 | 4.3 | 3.1 | 3.21 | 5.07 |

Figure 4. 5. Sediment export map of each land use t/ha/year

Similar to the result observed in the spatial distribution of soil loss, variation of sediment export was observed with different LULC classes. From the total sediment that reaches the surrounding water bodies, the highest contribution was from cultivated land 40.7% (Table 12). However, the highest sediment export per unit of area (ha) was observed also from cultivated land.

The estimated sediment export along the LULC classes was also in line with the results obtained in the spatial distribution of the soil loss. The result revealed that the highest contribution to the total sediment export comes from cultivated lands. The result conforms to the findings of (Nigussie Haregeweyn et al., 2015) who found higher sediment export from cultivated lands. This is may be likely associated with the cultivation of steep slopes, intensive plowing and mono cropping practices, and poor land management activities. Hence, the result gives a reason to suggest the need for promoting sustainable land management practices on steep slopes in the watershed.

Table4.10. The contributions of eache LULC type to the total sediment export in the study watershed during 1990-2020 periods.

| year | Sediment export (tones) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Cultivated land | Dense forest land | Grazing land | Open forest | Settlement land |  | total |
| 1990 | 445 | 80.5 | 72 | 124.4 | 22.5 |  | 744.4 |
| 2000 | 225.2 | 76.1 | 33 | 177 | 46.9 |  | 558.2 |
| 2010 | 271.2 | 42.65 | 6.9 | 72 | 25.76 |  | 418.51 |
| 2020 | 421.6 | 22.72 | 8.6 | 59.52 | 77.04 |  | 589.5 |
|  | Percent% |  |  |  |  |  |  |
| 1990 | 59.78 | 10.8 | 9.7 | 16.72 | 3 | 100 |  |
| 2000 | 40.35 | 13.63 | 5.91 | 31.71 | 8.4 | 100 |  |
| 2010 | 64.8 | 10.2 | 1.64 | 17.2 | 6.16 | 100 |  |
| 2020 | 71.5 | 3.85 | 1.45 | 10.1 | 13.1 | 100 |  |

## 4.5. Validation of model performance

The InVEST SDR model was validated by comparing the observed and simulated data of the sediment load. The obtained R2 value 0.79, 0.87 and 0.76 for validation period of 1990, 2000 and 2010 is very good consistency between the observed and simulated data while R2 value 0.69 for validation period 2020 periods show good consistency between the observed and simulated data, and indicates less error variance between the two data (Moriasi et al., 2007; Chaibou Begou et al., 2016; Gyamfi et al., 2016a; Tekalegn Ayele et al., 2017). PBAIS is 8.57, 17.24, 26.53 and 16.87 in 1990, 2000, 2010 and 2020 respectively which is very good performance in 1990 and good performance in 2000, 2010 and 2020 validation periode Moriasi et al. (2007). The value of RSR 0.5, 0.44, 0.18 and 0.46 the performance of the model is very good. In general, the overall performance indices obtained during the validation periods indicated a very good RSR and good R2 and PBIAS performancerate of the model the effects of LULC changes over the 1990 to 2020 periods.

Table4.11. Model performance statistics for the validation periods of 1990 to 2020

| Year | R2 | PBIAS | RSR |
| --- | --- | --- | --- |
| 1990 | 0.79 | 8.57 | 0.54 |
| 2000 | 0.87 | 17.24 | 0.44 |
| 2010 | 0.76 | 26.53 | 0.18 |
| 2020 | 0.69 | 16.89 | 0.46 |
| Overall performance | 0.78 | 17.3 | 0.4 |

# 5. CONCLUSION AND RECOMMENDATION

## 5.1. Conclusion

Land use and land cover change is a major cause of soil loss and sediment yield at watershed, regional and global scales. From the results of this study, it can be concluded that Maybar sub watershed had experienced a slight change in LULC over the past three decades. These changes in LULC have also affected the soil loss rate and sediment yield at the watershed although variations existed between the different decades. The result of land use land cover change shows that from 1990 to 2020, the proportion of dense forest and settlement land has increased slightly 19.8%, 25.2%, 36.68%, 39.1% and 12.9%, 18.4%, 19.3%, 20.7% during the study periods of 1990, 2000, 2010 and 202o respectively, while grassland and open forest land have decreased 10.32%, 5.5%, 2.58%, 1.7% and 31.47%, 27.5%, 20.64, 16.5% in 1990, 2000, 2010 and 2020 respectively. Besides, cultivated land has decreased 25.51%, 23.4% and 20.8% during 1990, 2000 and 2010 increased 22% during 2020.

The result indicated a temporal variation in soil erosion and sediment yield due to LULC change. The mean soil erosion decreased 28.9, 25.12 and 21.06 t/ha/years for 1990, 2000 and 2010 study years respectively while increased to 26.6 t/ha/years in 2020 years where as the mean sediment yield decrease 6.4, 4.8 and 3.6 t/ha/years for 1990, 2000 and 2010 study years and increased to 5.07 t/ha/years in 2020 study periods. Soil loss and sediment export have largely increased in the cultivated land while soil losses and sediment export in the dense forest and settlement land decreased.

## 5.2. Recommendation

According to the land use land cover change result cultivated land has increased in 2020. Soil erosion and sediment yield also showed an increasing in 2020. Therefore I would like to recommend that it should be considered and solve this problem.

- If the government concentrates on strengthening watershed institutions

- reduction in continuous cultivation in the higher altitude areas of the watershed overcome the challenges of dam siltation and ecosystem degradation in the lower part of the watershed

- Implementing proper soil and water conservation measures are highly necessary to arrest the expected increase of soil erosion and sediment yield in the study period of 2020.

- Policy instruments need to be enforced by making a buffer zone and implementing urgent conservation activities in the watershed to prevent soil loss and sediment export to the downward dams. .

- Experts of environment, conservation, resource management and sustainability, ecology, biodiversity, and eco-system are required to develop a plan for sustainable use of the site to ensure its functions for the next generations.

- Moreover, study on the effects of land use land cover change with associated soil erosion and sediment export is one of the future recommended research areas in this watershed.

# 6. REFERANCE

Abate Shiferaw, and Singh, K.L., 2010. An appraisal of the challenges that affect sustainability and productivity of the land use in the Borena Woreda of South Wollo highlands: Ethiopia. Journal of Sustainable Development in Africa, 12(6): pp.1520-5509.

Abate Shiferaw, and Singh, K.L., 2011. Evaluating the  use and  cover dynamics in Borena Woreda South Wollo Highs, Ethiopia. Journal of Business and Economics (The), 2(1).

Abate Shiferaw. (2011a). Estimating soil loss rates for soil conservation planning in the Borena woreda of South Wollo highs, Ethiopia. J Sustain Dev Afr. 13 (3):87-106.

Abbaspour, K.C., Yang, J., Maximov, I., Siber, R., Bogner, K., Mieleitner, J., Zobrist, J. and Srinivasan, R., 2007. Modelling hydrology and water quality in the pre-alpine/alpine Thur watershed using SWAT. Journal of hydrology, 333(2-4), pp.413-430.

Abd El-Kawy, O.R., Rød, J.K., Ismail, H.A. and Suliman, A.S., 2011.  use and  cover change detection in the western Nile delta of Egypt using remote sensing data. Applied geography, 31(2): pp.483-494.

Abdalla Abdelsalam Ahmed,  and Ismail, U.H.A.E., 2008.Sediment in the Nile River system Consultancy Study requested by UNESCO.

Abebe Semu Arayaselassie, 2018. The study of homegarden agrobiodiversity, practices of homegardening and its role for In-Situ conservation of plant biodiversity in Eastern Hararghe, Kombolcha Town Oromia Regional State Ethiopia. Open Journal of Forestry, 8(02): p.229.

Abineh Tilahun and Bogale Teferie 2015. Accuracy assessment of  use  cover classification using Google Earth. American Journal of Environmental Protection, 4(4): pp.193-198.

Abonesh, Tesfaye, Workneh Negatu, R. Brouwer, and P. Van der Zaag.2014. Understanding soil conservation decision of farmers in the Gedeb watershed, Ethiopia.  Degradation & Development, 25(1): pp.71-79.

Abreham Berta Aneseyee,, Teshome Soromessa, and Eyasu Elias.2020. The effect of  use/ cover changes on ecosystem services valuation of Winike watershed, Omo Gibe basin, Ethiopia. Human and Ecological Risk Assessment: An International Journal, 26(10): pp.2608-2627.

Adenew Taffa Ariti van Vliet, J. and Verburg, P.H., 2015. -use and -cover changes in the Central Rift Valley of Ethiopia: Assessment of perception and adaptation of stakeholders. Applied Geography, 65, pp.28-37.

Adornado, H.A., Yoshida, M. and Apolinares, H.A.,2009. Erosion vulnerability assessment in REINA, Quezon Province, Philippines with raster-based tool built within GIS environment. Agricultural Information Research, 18(1), pp.24-31.

Agarwal, C., 2002. A review and assessment of -use change models: dynamics of space, time, and human choice (Vol. 297).US Department of Agriculture, Forest Service, and Northeastern Research Station.

Agumassie Genet., 2020. Population growth and  use  cover change scenario in Ethiopia. International Journal of Environmental Protection and Policy, 8(4), pp.77-85. Genet, A., 2020. Population growth and  use  cover change scenario in Ethiopia. International Journal of Environmental Protection and Policy, 8(4), pp.77-85.

Ahmed Moustafa Moussa., 2018. Assessment of sediment deposition in Aswan high dam reservoir during 50 years (1964–2014). Grand Ethiopian Renaissance Dam Versus Aswan High Dam, pp.233-253.

Akalu Teshome, A., de Graaff, J., Ritsema, C. and Kassie, M., 2016. Farmers' perceptions about the influence of  quality,  fragmentation and tenure systems on sustainable  management in the north western Ethiopian highs.  degradation & development, 27(4), pp.884-898.

Aksoy, H. and Kavvas, M.L., 2005. A review of hillslope and watershed scale erosion and sediment transport models. Catena, 64(2-3), pp.247-271.

Aksoy, H., Mahe, G., Meddi, M., 2019. Modeling and Practice of Erosion and Sediment Transport under Change, 1–9

Alemayehu A Shawul, and Chakma, S., 2019. Spatiotemporal detection of  use/ cover change in the large basin using integrated approaches of remote sensing and GIS in the Upper Awash basin, Ethiopia. Environmental Earth Sciences, 78(5), pp.1-13.

Alexakis, D.D., Hadjimitsis, D.G. and Agapiou, A., 2013. Integrated use of remote sensing, GIS and precipitation data for the assessment of soil erosion rate in the catchment area of “Yialias” in Cyprus. Atmospheric Research, 131, pp.108-124.

Amare Bantider, Hurni, H and Gete Zeleke.,2011. Responses of rural households to the impacts of population and -use changes along the Eastern Escarpment of Wello, Ethiopia. Norsk Geografisk Tidsskrift-Norwegian Journal of Geography, 65(1), pp.42-53.

Amare Bantider., 2007. Landscape Transformation and Opportunities for Sustainable Land Management along the Eastern Escarpment of Wello (EEW), Ethiopia (Doctoral dissertation, Verlag nicht ermittelbar).

Amare Sewnet, 2015. Land use/cover change at infraz Watershed, Northwestren Ethiopia. Journal of Landscape Ecology, 8(1): pp.69-83.

Amare Sisay, Nega Chalie, Zenebe Girmay, Goitom Takele, and Alemayoh Tolera 2014. Landscape-scale soil erosion modeling and risk mapping of mountainous areas in eastern escarpment of Wondo Genet Watershed, Ethiopia. Int Res J Agric Sci Soil Sci, 4(6), pp.107-116.

Ambers, R.K., 2001. Using the sediment record in a western Oregon flood-control reservoir to assess the influence of storm history and logging on sediment yield. Journal of Hydrology, 244(3-4), pp.181-200.

Angima, S.D., Stott, D.E., O’neill, M.K., Ong, C.K. and Weesies, G.A., 2003. Soil erosion prediction using RUSLE for central Kenyan highland conditions. Agriculture, ecosystems & environment, 97(1-3), pp.295-308.

Arega Degife, Hailu Worku, and Shumete Gizaw, 2021. Environmental implications of soil erosion and sediment yield in Lake Hawassa watershed, south-central Ethiopia. Environmental Systems Research, 10, pp.1-24.

Arega Degife, Hailu Worku, Shumete Gizaw, and Abiyot Legesse., 2019. Land use land cover dynamics, its drivers and environmental implications in Lake Hawassa Watershed of Ethiopia. Remote sensing applications: society and environment, 14, pp.178-190.

Arnold, J.G., Moriasi, D.N., Gassman, P.W., Abbaspour, K.C., White, M.J., Srinivasan, R., Santhi, C., Harmel, R.D., Van Griensven, A., Van Liew, M.W. and Kannan, N., 2012. SWAT: Model use, calibration, and validation. Transactions of the ASABE, 55(4), pp.1491-1508.

Arnold, J.G., Moriasi, D.N., Gassman, P.W., Abbaspour, K.C., White, M.J., Srinivasan, R., Santhi, C., Harmel, R.D., Van Griensven, A., Van Liew, M.W. and Kannan, N., 2012. SWAT: Model use, calibration, and validation. Transactions of the ASABE, 55(4), pp.1491-1508.

Arnold, J.G., Srinivasan, R., Muttiah, R.S. and Allen, P.M., 1999. Continental scale simulation of the hydrologic balance 1. JAWRA Journal of the American Water Resources Association, 35(5), pp.1037-1051.

Ashenafi Burka., 2008. Land Use/Land Cover Dynamics in Prosopis juliflora invaded area of Metehara and the Surrounding Districts Using Remote Sensing and GIS Techniques. Addis Ababa University, Ethiopia.

Ashiagbor, G., Forkuo, E.K., Laari, P. and Aabeyir, R., 2013. Modeling soil erosion using RUSLE and GIS tools. Int J Remote Sens Geosci, 2(4), pp.1-17.

Asmamaw, L.B., Mohammed, A.A. and Lulseged, T.D., 2011. Land use/cover dynamics and their effects in the Gerado catchment, northeastern Ethiopia. International Journal of Environmental Studies, 68(6), pp.883-900.

Asnake Yimam Yesuph and Amare Bantider Dagnew., 2019. Land use/cover spatiotemporal dynamics, driving forces and implications at the Beshillo catchment of the Blue Nile Basin, North Eastern Highlands of Ethiopia. Environmental Systems Research, 8(1), pp.1-30.

Asnake Yimam Yesuph,, and Amare Bantider Dagnew 2019. Soil erosion mapping and severity analysis based on RUSLE model and local perception in the Beshillo Catchment of the Blue Nile Basin, Ethiopia. Environmental Systems Research, 8(1), pp.1-21. .

Asnake, Mekuriaw 2017. Assessing the effectiveness of land resource management practices on erosion and vegetative cover using GIS and remote sensing techniques in Melaka watershed, Ethiopia. Environmental Systems Research, 6(1): pp.1-10.

Awdenegest Moges and Holden, N.M., 2007. Farmers' perceptions of soil erosion and soil fertility loss in Southern Ethiopia. Land Degradation & Development, 18(5), pp.543-554. Link: https://tinyurl.com/y3yfmelt.

Babiso, B., Toma, S. and Bajigo, A., 2016. Land use/land cover dynamics and its implication on sustainable land management in Wallecha watershed, southern Ethiopia. Global Journal of Science Frontier Research: H Environment & Earth Science, 16(4), pp.49-53.

Baral, H., Keenan, R.J., Sharma, S.K., Stork, N.E. and Kasel, S., 2014. Spatial assessment and mapping of biodiversity and conservation priorities in a heavily modified and fragmented production landscape in north-central Victoria, Australia. Ecological Indicators, 36, pp.552-562.

Basin, M., 2019. Finite-and fixed-time convergent algorithms: Design and convergence time estimation. Annual reviews in control, 48, pp.209-221.

Begou, T., Lemarchand, F. and Lumeau, J., 2016. Advanced optical interference filters based on metal and dielectric layers. Optics Express, 24(18), pp.20925-20937.

Bekele Tsegaye., 2019. Effect of land use and land cover changes on soil erosion in Ethiopia. International Journal of Agricultural Science and Food Technology, 5(1), pp.026-034.

Belay Tegene. 2002. Land-cover/land-use changes in the derekolli catchment of the South Welo Zone of Amhara Region, Ethiopia. Eastern Africa Social Science Research Review 18(1)  pp.1-20.

Belayneh Yigez, Xiong, D., Zhang, B., Yuan, Y., Baig, M.A., Dahal, N.M., Guadie, A., Zhao, W. and Wu, Y., 2021. Spatial distribution of soil erosion and sediment yield in the Koshi River Basin, Nepal: a case study of Triyuga watershed. Journal of Soils and Sediments, 21(12), pp.3888-3905.

Belshaw, R.L. and Jerram, D.L., 1986. Garments designed to reduce fire hazard. Fire safety journal, 10(1), pp.19-28.

Benaud, P., Anderson, K., Evans, M., Farrow, L., Glendell, M., James, M.R., Quine, T.A., Quinton, J.N., Rawlins, B., Rickson, R.J. and Brazier, R.E., 2020. National-scale geodata describe widespread accelerated soil erosion. Geoderma, 371, p.114378.

Benavidez R, Jackson B, Maxwell D, Norton K (2018) A review of the (Revised) Universal Soil Loss Equation ((R) USLE): with a view to increasing its global applicability and improving soil loss estimates. Hydrol Earth Syst Sci 22(11): pp 6059–6086.

Biggelaar, C., Lal, R., Eswaran, H., Breneman, V.E. and Reich, P.F., 2003. Crop yield losses to soil erosion at regional and global scales: Evidence from plot-level and GIS data. In Land Quality, Agricultural Productivity, and Food Security (pp. 262-279). Edward Elgar Publishing

Binyam Alemu, Efrem Garedew, Zewdu Eshetu, and Habtemariam Kassa.2015. Land use and land cover changes and associated driving forces in north western lowlands of Ethiopia. International research journal of agricultural science and soil science, 5(1), pp.28-44.

Blanco H and Lal R (2008). Principles of Soil Conservation and Management. Springer Science+Business Media B.V. ISBN: 978-1- 4020-8708. 240pp.

Boakye, E., Anyemedu, F.O.K., Donkor, E.A. and Quaye-Ballard, J.A., 2020. Spatial distribution of soil erosion and sediment yield in the Pra River Basin. SN Applied Sciences, 2(3), pp.1-12.

Borrelli, P., Robinson, D.A., Fleischer, L.R., Lugato, E., Ballabio, C., Alewell, C., Meusburger, K., Modugno, S., Schütt, B., Ferro, V. and Bagarello, V., 2017. An assessment of the global impact of 21st century land use change on soil erosion. Nature communications, 8(1): pp.1-13.

Borselli, L., Cassi, P. and Torri, D., 2008. Prolegomena to sediment and flow connectivity in the landscape: a GIS and field numerical assessment. Catena, 75(3), pp.268-277.

Borselli, L., Torri, D., Poesen, J. and Iaquinta, P., 2012. A robust algorithm for estimating soil erodibility in different climates. Catena, 97, pp.85-94.

Boskidis, I., Gikas, G., Sylaios, G., and Tsihruntzis, V.A. (2012). “Hydrologic and water quality modelling of lower Nestos river basin”. Water Resour. Manage. 26(10), 3023–3051. doi: 10.1007/ s11269-012-0064-7.

Bosshart, U., 1997. Photo-monitoring. Soil Conservation Research Programme.

Boughton, W.C., 1989. A review of the USDA SCS curve number method. Soil Research, 27(3), pp.511-523.

Briassoulis, D., Babou, E. and Hiskakis, M., 2011. Degradation behaviour and field performance of experimental biodegradable drip irrigation systems. Journal of Polymers and the Environment, 19(2), pp.341-361.

Brook Lemma, 2004, March. Human intervention in two lakes: Lessons from Lakes Alemaya and Hora-Kilole. In Proceedings of the National Consultative workshop on the Ramsar Convention and Ethiopia (pp. 18-19).

Cavalli M, Trevisani S, Comiti F, Marchi L (2013) Geomorphometric assessment of spatial sediment connectivity in small Alpine catchments. Geomorphology 188:31–41.

Cerdan, O., Govers, G., Le Bissonnais, Y., Van Oost, K., Poesen, J., Saby, N., Gobin, A., Vacca, A., Quinton, J., Auerswald, K. and Klik, A., 2010. Rates and spatial variations of soil erosion in Europe: A study based on erosion plot data. Geomorphology, 122(1-2), pp.167-177.

Chaibou Begou, J., Jomaa, S., Benabdallah, S., Bazie, P., Afouda, A. and Rode, M., 2016. Multi-site validation of the SWAT model on the Bani catchment: Model performance and predictive uncertainty. Water, 8(5), p.178.

Chu, Z.X., Zhai, S.K., Lu, X.X., Liu, J.P., Xu, J.X., Xu, K.H., 2009 A quantitative assessment of human impacts on decrease in sediment flux from major Chinese rivers entering the western Pacific Ocean. Geophys. Res.Lett.

Constable, M and Belshaw, D., 1986. A summary of major findings and recommendations from the Ethiopian higlands reclamation study (ETHRS).

Dabral, P.P., Baithuri, N. and Pandey, A., 2008. Soil erosion assessment in a hilly catchment of North Eastern India using USLE, GIS and remote sensing. Water Resources Management, 22(12), pp.1783-1798.

De Asis, A.M. and Omasa, K., 2007. Estimation of vegetation parameter for modeling soil erosion using linear Spectral Mixture Analysis of Landsat ETM data. ISPRS Journal of Photogrammetry and Remote Sensing, 62(4), pp.309-324.

De Vente, J., Poesen, J., Verstraeten, G., Govers, G., Vanmaercke, M., Van Rompaey, A., Arabkhedri, M. and Boix-Fayos, C., 2013. Predicting soil erosion and sediment yield at regional scales: where do we stand?. Earth-Science Reviews, 127, pp.16-29.

Degefie, Tibebe, and Woldeamlak Bewket 2011. Surface runoff and soil erosion estimation using the SWAT model in the Keleta watershed, Ethiopia. Land Degradation & Development, 22(6), pp.551-564.

Delalay, M., Tiwari, V., Ziegler, A.D., Gopal, V. and Passy, P., 2019. Land-use and land-cover classification using Sentinel-2 data and machine-learning algorithms: operational method and its implementation for a mountainous area of Nepal. Journal of Applied Remote Sensing, 13(1), pp.014530-014530.

Derege Tsegaye Meshesha, Tsunekawa, A., Tsubo, M. and Nigussie Haregeweyn., 2012. Dynamics and hotspots of soil erosion and management scenarios of the Central Rift Valley of Ethiopia. International Journal of Sediment Research, 27(1), pp.84-99.

Desalew Meseret Moges and Bhat, H.G.2017. Integration of geospatial technologies with RUSLE for analysis of land use/cover change impact on soil erosion: case study in Rib watershed, north-western highland Ethiopia. Environmental earth sciences, 76(22), pp.1-14.

Dessie, Assefa, Rewald, B., Sandén, H., Rosinger, C, Abrham Abiyu, Birru Yitaferu, and Godbold, D.L., 2017. Deforestation and land use strongly effect soil organic carbon and nitrogen stock in Northwest Ethiopia. Catena, 153, pp.89-99.

Diress Tsegaye, Moe, S.R., Vedeld, P. and Aynekulu, E.,2010. Land-use/cover dynamics in Northern Afar rangelands, Ethiopia. Agriculture, ecosystems & environment, 139(1-2), pp.174-180.

Dubale, P., 2001. Soil and water resources and degradation factors affecting productivity in Ethiopian highland agro-ecosystems. Northeast African Studies, pp.27-51.

Easton, Z.M., Fuka, D.R., White, E.D., Collick, A.S., Biruk Ashagre, B., McCartney, M., Awulachew, S.B., Ahmed, A.A. and Steenhuis, T.S., 2010. A multi basin SWAT model analysis of runoff and sedimentation in the Blue Nile, Ethiopia. Hydrology and earth system sciences, 14(10), pp.1827-1841.

Ebrahim Esa Hassen and Mohamed Assen.,2018. Land use/cover dynamics and its drivers in Gelda catchment, Lake Tana watershed, Ethiopia. Environmental Systems Research, 6(1), pp.1-13.

Ermias Teferi , S. Uhlenbrook, Woldeamlak Bewket, J. Wenninger, and Belay Simane., 2010. The use of remote sensing to quantify wetland loss in the Choke Mountain range, Upper Blue Nile basin, Ethiopia. Hydrology and Earth System Sciences, 14(12): pp.2415-2428.

Ermias Teferi, Woldeamlak Bewket, and Belay Simane., 2016. Effects of land use and land cover on selected soil quality indicators in the headwater area of the Blue Nile basin of Ethiopia. Environmental monitoring and assessment, 188(2), pp.1-12.

Eron, J.J., Clotet, B., Durant, J., Katlama, C., Kumar, P., Lazzarin, A., Poizot-Martin, I., Richmond, G., Soriano, V., Ait-Khaled, M. and Fujiwara, T., 2013. Safety and efficacy of dolutegravir in treatment-experienced subjects with raltegravir-resistant HIV type 1 infection: 24-week results of the VIKING Study. The Journal of infectious diseases, 207(5), pp.740-748.

Eshetu Tufa Senti, Bayissa Waltaji Tufa, and Kbrom Ambachew Gebrehiwot.2014. Soil erosion, sediment yield and conservation practices assessment on Lake Haramaya Catchment. World Journal of Agricultural Sciences, 2(7), pp.186-193.

FAO (1986) Ethiopian highlands reclamation study, Ethiopia. Final Report. FAO, Rome

FAO (2015) Soil change: Impacts and responses. ISBN 978-92-5-109004-6

FAO, “Food and Agriculture Organization Corporate Statistical Database (FAOSTAT),” 2020, http://www.fao.org/ faostat/en/ data.

Farhan, Y. and Nawaiseh, S., 2015. Spatial assessment of soil erosion risk using RUSLE and GIS techniques. Environmental Earth Sciences, 74(6): pp.4649-4669.

Fekadu Yohannes, 2005, September. Soil erosion and sedimentation: The case of Lake Alemaya. In Proceedings of the Second Awareness Creation Workshop on Wetlands in the Amhara Region, Bahirdar, Ethiopia.

Fikir Alemayehu Nurhussen Taha, Nyssen,J, Atkilt Girma, Amanuel Zenebe, Mintesinot Behailu Deckers, S. and Poesen, J., 2009. The impacts of watershed management on land use and land cover dynamics in Eastern Tigray (Ethiopia). Resources, Conservation and Recycling, 53(4), pp.192-198.

Fikirte Demissie, Kumelachew Yeshitila, Mengistie Kinduand Schneider, T., 2017. Land use/Land cover changes and their causes in Libokemkem District of South Gonder, Ethiopia. Remote Sensing Applications: Society and Environment, 8, pp.224-230.

Fikre Zerfu, Abdurehman Mektel, and Biniyam Bogale.., 2019. Land use and land cover dynamics in the North-eastern Somali rangelands of eastern Ethiopia. International Journal of Geosciences, 10(09), p.811.

Fiseha, Behulu M., S. G. Setegn, A. M. Melesse, E. Volpi, and A. Fiori. "  2014. Impact of climate change on the hydrology of upper Tiber River Basin using bias corrected regional climate model. Water Resources Management, 28, pp.1327-1343

Ganasri, B.P. and Ramesh, H., 2016. Assessment of soil erosion by RUSLE model using remote sensing and GIS-A case study of Nethravathi Basin. Geoscience Frontiers, 7(6), pp.953-961.

Gao, J., Li, F., Gao, H., Zhou, C. and Zhang, X., 2017. The effect of changes in land-use on water-related ecosystem services: a study of the Guishui River Basin, Beijing, China. Journal of Cleaner Production, 163, pp.S148-S155.

Gebrekidan, Worku, Amare Bantider, and Habtamu Temesgen., 2014. Land use and land cover change in Ameleke Watershed, South Ethiopia. Journal of Natural Sciences Research, 4(14):  pp.42-47.

Gebremedhin Tesfaye, Abera Assefa and Desale Kidane.2017. Runoff, sediment load and land use/cover change relationship: the case of Maybar sub-watershed, South Wollo, Ethiopia. International journal of river basin management, 15(1), pp.89-101.

Gebremicael, T.G., Mohamed, Y.A., Betrie, G.D., Van der Zaag, P. and Teferi, E., 2013. Trend analysis of runoff and sediment fluxes in the Upper Blue Nile basin: A combined analysis of statistical tests, physically-based models and landuse maps. Journal of Hydrology, 482:  pp.57-68.

Gebreyesus, Brhane, and Kirubel Mekonen. 2009. Estimating soil loss using Universal Soil Loss Equation (USLE) for soil conservation planning at Medego watershed, Northern Ethiopia. Journal of American Science, 5(1), pp.58-69.

Geist, H.J. and Lambin, E.F., 2002. Proximate Causes and Underlying Driving Forces of Tropical DeforestationTropical forests are disappearing as the result of many pressures, both local and regional, acting in various combinations in different geographical locations. BioScience, 52(2), pp.143-150.

Gelagay, H.S., 2016. RUSLE and SDR model based sediment yield assessment in a GIS and remote sensing environment; a case study of Koga watershed, Upper Blue Nile Basin, Ethiopia. Hydrology Current Research, 7(2), p.239.

Gete .Zeleke, 2000. Landscape Dynamics and Soil Erosion Process Modeling in the North-western Ethiopian Highlands. Thesis (PhD). Center for Development and Environment.

Gete Zeleke and Hurni H (2001). Implications of land use and land cover dynamics for mountain resource degradation in the Northwestern Ethiopian Highlands. Mountain Research and Development. 21 (2): 184-191.

Ghoraba, S.M., 2015. Hydrological modeling of the Simly Dam watershed (Pakistan) using GIS and SWAT model. Alexandria Engineering Journal, 54(3), pp.583-594.

Girma Ayele, Hussein Hayicho, and Mersha Alemu., 2019. Land use land cover change detection and deforestation modeling: in Delomena District of Bale Zone, Ethiopia. Journal of Environmental Protection, 10(4), pp.532-561.

Githui, F., Gitau, W., Mutua, F. and Bauwens, W., 2009. Climate change impact on SWAT simulated streamflow in western Kenya. International Journal of Climatology: A Journal of the Royal Meteorological Society, 29(12), pp.1823-1834.

Githui, F., Mutua, F. and Bauwens, W., 2009. Estimating the impacts of land-cover change on runoff using the soil and water assessment tool (SWAT): case study of Nzoia catchment, Kenya/Estimation des impacts du changement d'occupation du sol sur l'écoulement à l'aide de SWAT: étude du cas du bassin de Nzoia, Kenya. Hydrological sciences journal, 54(5), pp.899-908.

Guo, B., Yang, F., Fan, J., Lu, Y., 2022. The changes of spatiotemporal pattern of rocky desertification and its dominant driving factors in typical karst mountainous areas under the background of global change. Rem. Sens. 14 (10), 1–24.

Gupta, H.V., Sorooshian, S. and Yapo, P.O., 1999. Status of automatic calibration for hydrologic models: Comparison with multilevel expert calibration. Journal of hydrologic engineering, 4(2), pp.135-143.

Gyamfi, C., Ndambuki, J.M. and Salim, R.W., 2016. Application of SWAT model to the Olifants Basin: calibration, validation and uncertainty analysis. Journal of Water Resource and Protection, 8(03), p.397.

Habtamu Sewnet., Amare Sewnet. (2016). Soil loss estimation using GIS and remote sensing techniques: A case of Koga watershed, Northwestern Ethiopia. Int. Soil Water Conserv Res. http://dx.doi.org/10.1016/j.iswcr.2016.01.002i

Habtamu Temesgen, Nyssen, J., Amanuel Zenebe, Nigussie Haregeweyn, Mengistie Kindu, Mulugeta Lemenih, and Mitiku Haile., 2013. Ecological succession and land use changes in a lake retreat area (Main Ethiopian Rift Valley). Journal of Arid Environments, 91, pp.53-60.

Habtamu, Sewnet Gelagay 2016. RUSLE and SDR model based sediment yield assessment in a GIS and remote sensing environment; a case study of Koga watershed, upper Blue Nile basin, Ethiopia. Hydrology: Current Research, 7(2).

Hagos Gebreslassie., 2014. Land use-land cover dynamics of Huluka watershed, Central Rift Valley, Ethiopia. International Soil and Water Conservation Research, 2(4), pp.25-33.

Haile E., Getachew and Assefa M. Melesse, 2012. The impact of land use change on the hydrology of the Angereb Watershed, Ethiopia. International Journal of Water Sciences, 1(6).

Haile Mitiku,., A. Tedros, K. Witten, Y. Mekonnen, P. Byass, and P. Lindsay.., 2002, July. Environmental and social aspects of earth dam building in the semi-arid areas of Tigray, Ethiopia. In 6th symposium on Sustainable Water Resources Development (pp. 8-9).

Haile. Mitiku, Herweg, K.G. and Stillhardt, B., 2006. Sustainable land management: a new approach to soil and water conservation in Ethiopia.

Hamel, P., Chaplin-Kramer, R., Sim, S. and Mueller, C., 2015. A new approach to modeling the sediment retention service (InVEST 3.0): Case study of the Cape Fear catchment, North Carolina, USA. Science of the Total Environment, 524, pp.166-177. Hamel, P., Chaplin-Kramer, R., Sim, S. and Mueller, C., 2015. A new approach to modeling the sediment retention service (InVEST 3.0): Case study of the Cape Fear catchment, North Carolina, USA. Science of the Total Environment, 524, pp.166-177.

Hamel, P., Falinski, K., Sharp, R., Auerbach, D.A., Sánchez-Canales, M. and Dennedy-Frank, P.J., 2017. Sediment delivery modeling in practice: Comparing the effects of watershed characteristics and data resolution across hydroclimatic regions. Science of the Total Environment, 580, pp.1381-1388.

Hamere, Yohannes,  Teshome Soromessa, Mekuria Argaw and Dewan, A., 2021. Impact of landscape pattern changes on hydrological ecosystem services in the Beressa watershed of the Blue Nile Basin in Ethiopia. Science of the Total Environment, 793, p.148559.

Han, J., Ge, W., Hei, Z., Cong, C., Ma, C., Xie, M., Liu, B., Feng, W., Wang, F. & Jiao, J. 2020 Agricultural land use and management weaken the soil erosion induced by extreme rainstorms. Agriculture, Ecosystems and Environment 301, 107047.

Hans, H.U.R.N.I., 2002. Current international actions for furthering the sustainable use of soils. In 17. World congress of soil science,, Bangkok (Thailand), 14-21 Aug 2002.

Hayal desta and brook lemma. 2017. SWAT based hydrological assessment and characterization of Lake Ziway sub-watersheds, Ethiopia. Journal of Hydrology: Regional Studies, 13, pp.122-137.

Hurni H (1985) Erosion-productivity-conservation systems in Ethiopia. In: proceedings of paper presented at the 4th international conference on soil conservation, Maracay, Venezuela

Hurni H (1989) Applied soil conservation research in Ethiopia. In: Thomas, DB (ed.) Soil and water conservation in Kenya. Nairobi, Kenya: University of Nairobi; Swedish International Development Authorit. 5-21. Link: https://tinyurl.com/yy59kfad.

Hurni H (1990) Degradation and conservation of soil resources in the Ethiopian highlands. Mountain Research and Development 8: 123-130. Link: https://tinyurl.com/yxmjgyql.

Hurni H (1993) Land degradation, famine, and land resource scenarios in Ethiopia. In: Pimentel, D. (Ed.) World Soil Erosion and Conservation, Cambridge. Cambridge University Press. Green Growth 27–62. Link: https://tinyurl.com/y5jheypo.

Hurni H (2002). Current international actions for furthering the sustainability use of soils. Symposium paper no.63 on 17th WCSS, 14- 21 August 2002. 1-8 pp.

Hurni H, Kebede Tato and Gete Zeleke 2005. The implications of changes in population, land use, and land management for surface runoff in the upper Nile basin area of Ethiopia. Mountain research and development, 25(2): pp.147-154.

Hurni, H, 1988. Degradation and conservation of the resources in the Ethiopian highlands. Mountain research and development, pp.123-130.

Hurni, H., 1983. Soil formation rates in Ethiopia. Ethiopian highlands reclamation study. Soil Conservation Research Project, FAO, UTF/ETH/037/ETH Working Paper 2.

Hurni, H., 1986. Guidelines for development agents on soil conservation in Ethiopia. Soil conservation research project, Community Forests and Soil Conservation Development Department, Ministry of Agriculture, Addis Ababa, Ethiopia.

Hurni, H., Kebede Tato, 1992. Soil conservation for survival. Soil and Water Conservation Society (SWCS).

Hurni, k, Gete Zeleke, Menale Kassie, Berhan Tegegne, Tibebu Kassawmar, Ermias Teferi, Aderajew Moges., 2015. Economics of Land Degradation (ELD) Ethiopia Case Study: Soil degradation and sustainable land management in the rainfed agricultural areas of Ethiopia: An assessment of the economic implications94 Link: https://tinyurl.com/y2vqcjd2. .

Jacob, M., Frankl, A., Beeckman, H., Gebrekidan Mesfin Hendrickx, M., Etefa Guyassa. and Nyssen, J., 2015. North Ethiopian afro‐alpine tree line dynamics and forest‐cover change since the early 20th century. Land Degradation & Development, 26(7):  pp.654-664.

Jahun, B.G., Ibrahim, R., Dlamini, N.S. and Musa, S.M., 2015. Review of soil erosion assessment using RUSLE model and GIS. J Biol Agric Healthc, 5(9), pp.36-47. Research

Jansson, M.B., 1988. A global survey of sediment yield. Geografiska Annaler: Series A, Physical Geography, 70(1-2), pp.81-98.

Jensen, J. R. (2005). Introductory digital image processing: A remote sensing perspective (John R. Jensen, Ed.) (3rd ed.). Pearson Education Inc.

Joint FAO/WHO 2006. Expert Committee on Food Additives. Meeting and World Health Organization, Safety evaluation of certain contaminants in food (Vol. 82). Food & Agriculture Org.

Karaburun A (2010) Estimation of C factor for soil erosion modeling using NDVI in Buyukcekmece watershed. Ozean J Appl Sci 3(1): pp 77–85.

Kayet, N., Pathak, K., Chakrabarty, A. and Sahoo, S., 2018. Evaluation of soil loss estimation using the RUSLE model and SCS-CN method in hillslope mining areas. International Soil and Water Conservation Research, 6(1), pp.31-42.

Kebede Wolka, 2014. Effect of soil and water conservation measures and challenges for its adoption: Ethiopia in focus. Journal of Environmental science and Technology, 7(4):  pp.185-199.

Kebede, megersa, ankit chakravarti, and tamene adugna. 2017. Stream flow and land use land cover change in finchaa hydropower, blue nile basin, ethiopia."

Kebede, Wolka Wolancho., 2012. Watershed management: an option to sustain dam and reservoir function in Ethiopia. Journal of environmental science and technology, 5(5), pp.262-273.

Kebede, Wolka, Habitamu Tadesse, Efrem Garedew, and Fantaw Yimer2015. Soil erosion risk assessment in the Chaleleka wetland watershed, Central Rift Valley of Ethiopia. Environmental Systems Research, 4(1), pp.1-12.

Kiani-Harchegani, M., Sadeghi, S. H., Singh, V. P., Asadi, H. & Abedi, M. 2019 Effects of rainfall intensity and slope on sediment size distribution during erosion using partial eta squared. Catena 176, 65–72

Kindiye Ebabu, Tsunekawa, A Nigussie Haregeweyn, Enyew Adgo, Derege Tsegaye Meshesha, Dagnachew Aklog and Yibeltal, M., 2019. Effects of land use and sustainable land management practices on runoff and soil loss in the Upper Blue Nile basin, Ethiopia. Science of the Total Environment, 648: pp.1462-1475.

Kiros Tsegay Deribew and Desalegn Wana Dalacho.,2019. Land use and forest cover dynamics in the North-eastern Addis Ababa, central highlands of Ethiopia. Environmental Systems Research, 8(1), pp.1-18.

Kirui, O.K. and Mirzabaev, A., 2014. Economics of land degradation in Eastern Africa (No. 128). ZEF working paper series.

Klik, A., Kendie, H., Strohmeier, S., Schuster, G., Nachtnebel, H.P. and Ziadat, F., 2015. Assessment of current land use and potential soil and water conservation measures on surface run-off and sediment yield. Mitigating Land Degradation and Improving Livelihoods: An Integrated Watershed Approach, p.110.

Kouli, M., Soupios, P. and Vallianatos, F., 2009. Soil erosion prediction using the revised universal soil loss equation (RUSLE) in a GIS framework, Chania, Northwestern Crete, Greece. Environmental geology, 57(3), pp.483-497.

Kushwaha, A. and Jain, M.K., 2013. Hydrological simulation in a forest dominated watershed in Himalayan region using SWAT model. Water resources management, 27, pp.3005-3023.

Lambin, E.F. and Geist, H.J. eds., 2008.Land-use and land-cover change: local processes and global impacts. Springer Science & Business Media Gebreyesus Brhane and Kirubel Mekonen.2009. Estimating soil loss using Universal Soil Loss Equation (USLE) for soil conservation planning at Medego watershed, Northern Ethiopia. Journal of American Science, 5(1): pp.58-69.

Lambin, E.F. and Linderman, M., 2006. Time series of remote sensing data for land change science. IEEE transactions on geoscience and remote sensing, 44(7), pp.1926-1928.

Lambin, E.F., 2001. Global land-use and land-cover change: what have we learned so far? Global Change News, 46:  pp.27-30.

Lambin, E.F., Geist, H.J. and Lepers, E., 2003. Dynamics of land-use and land-cover change in tropical regions. Annual review of environment and resources, 28(1), pp.205-241.

Legates, D.R. and McCabe Jr, G.J., 1999. Evaluating the use of “goodness‐of‐fit” measures in hydrologic and hydroclimatic model validation. Water resources research, 35(1), pp.233-241.

Li J, Heap AD (2008) a review of spatial interpolation methods for environmental scientists. Geoscience Australia 2008/20

Liaqat, Muhammad Usman, Mohamed Mostafa Mohamed, Chowdhury, R., Elmahdy, S.I., Khan, Q. and Ansari, R 2021. Impact of land use/land cover changes on groundwater resources in Al Ain region of the United Arab Emirates using remote sensing and GIS techniques. Groundwater for Sustainable Development, 14, p.10058

Lillesand, T. M., Kiefer, R. W., & Chip Man, J. (2004). Remote sensing and image interpretation (J. Wiley, Ed.) (5th ed.).

Lin D, Gao Y, Wu Y, Shi P, Yang H, Wang JA (2017) A conversion method to determine the regional vegetation cover factor from standard plots based on large sample theory and TM images: a case study in the eastern farming-pasture ecotone of northern China. Remote Sensing 9(10): pp 1035.

Liu, C., Sui, J., He, Y., Hir shfield, F., 2013 Changes in runoff and sediment load from major Chinese rivers to the Pacific Ocean over the period 1955–2010. Int. J. Sediment Res. 28, 486–495. https://doi.org/10.1016/S1001-6279.

López-Vicente M, Poesen J, Navas A, Gaspar L (2013) Predicting runof and sediment connectivity and soil erosion by water for diferent land use scenarios in the Spanish Pre-Pyrenees. CATENA 102:62–73.

Lu, D., & Weng, Q. A. (2007). Survey of image classification methods and techniques for improving classification performance. International Journal of Remote Sensing, 28: pp. 823–870.

Lulseged Tamene and Quang Bao Le., 2015. Estimating soil erosion in sub-Saharan Africa based on landscape similarity mapping and using the revised universal soil loss equation (RUSLE). Nutrient cycling in agroecosystems, 102, pp.17-31.

Magliocca, N.R., Rudel, T.K., Verburg, P.H., McConnell, W.J., Mertz, O., Gerstner, K., Heinimann, A. and Ellis, E.C., 2015. Synthesis in land change science: methodological patterns, challenges, and guidelines. Regional environmental change, 15, pp.211-226.

Maliqi, E. and Singh, S.K., 2019. Quantitative estimation of soil erosion using open-access earth observation data sets and erosion potential model. Water Conservation Science and Engineering, 4, pp.187-200.

Manandhar, R., Odeh, I.O. and Ancev, T., 2009. Improving the accuracy of land use and land cover classification of Landsat data using post-classification enhancement. Remote Sensing, 1(3): pp.330-344.

Marondedze, A.K. and Schütt, B., 2020. Assessment of soil erosion using the RUSLE model for the Epworth District of the Harare Metropolitan Province, Zimbabwe. Sustainability, 12(20), p.8531.

Melkamu Bezabih,  Duncan, A.J.Aberra Adie, Kindu Mekonnen Khan, N.A. and Thorne, P, 2016. The role of irrigated fodder production to supplement the diet of fattening sheep by smallholders in southern Ethiopia Tropical and Subtropical Agro ecosystems, 19(3): pp.263-275.

Menale Kassie,  Köhlin, G., Bluffstone, R. and Holden, S., 2011, May. Are soil conservation technologies “win‐win?” A case study of Anjeni in the north‐western Ethiopian highlands. In natural resources forum (Vol. 35, No. 2, pp. 89-99). Oxford, UK: Blackwell Publishing Ltd Link: https://tinyurl.com/y4qazzaj.

Mengesha Zerihun,  Mohammed S. Mohammedyasin, Demeke Sewnet, Anwar A. Adem, and Mindesilew Lakew 2018. Assessment of soil erosion using RUSLE, GIS and remote sensing in NW Ethiopia. Geoderma regional, 12, pp.83-90

Mesfin Anteneh Wubie, Mohammed Assen and Nicolau, M.D., 2016. Patterns, causes and consequences of land use/cover dynamics in the Gumara watershed of lake Tana basin, Northwestern Ethiopia. Environmental Systems Research, 5(1), pp.1-12.

Mesfin, Sahle,  Osamu SaitoFürst, C and Kumelachew Yeshitela 2019. Quantifying and mapping of water-related ecosystem services for enhancing the security of the food-water-energy nexus in tropical data–sparse catchment. Science of the Total Environment, 646, pp.573-586.

Mesfine Abebaw, 2020. Effect of land use land cover changes on farmers’ income in farta woreda, north westethiopa  (doctoral dissertation).

Meyer, W.B. and Turner, B.L., 1992. Human population growth and global land-use/cover change. Annual review of ecology and systematics, pp.39-61.

Mitiku, Haile, Herweg, K.G. and Stillhardt, B., 2006. Sustainable land management: a new approach to soil and water conservation in Ethiopia.

Mohajane, M., Essahlaoui, A., Oudija, F., Hafyani, M.E., Hmaidi, A.E., Ouali, A.E., Randazzo, G. and Teodoro, A.C., 2018. Land use/land cover (LULC) using landsat data series (MSS, TM, ETM+ and OLI) in Azrou Forest, in the Central Middle Atlas of Morocco. Environments, 5(12), p.131.

Monserud, R. (1990). Methods for comparing global vegetation maps, Report WP-90-40, IIASA, Laxenburg.

Moore, I.D. and Burch, G.J. 1986a. Modeling erosion and deposition: topographic eﬀects. Trans. Asae 26, 1624–1630.

Moore, I.D. and Burch, G.J. 1986b. Physical basis of the length-slope factor in the universal soil loss equation. Soil Sci. Soc. Am. J. 50, 1294–1298

Morgan, R.P.C. (2005). Soil Erosion and Conservation, 3rd ed.; Blackwell Pub: Malden, MA, USA, 2005; ISBN 978-1-4051-1781-4.

Morgan, R.P.C., 2009 Soil erosion and conservation John Wiley & Sons.

Moriasi, D.N., Arnold, J.G., Van Liew, M.W., Bingner, R.L., Harmel, R.D. and Veith, T.L., 2007. Model evaluation guidelines for systematic quantification of accuracy in watershed simulations. Transactions of the ASABE, 50(3), pp.885-900

Morie, J.F., 2007. Performing in (virtual) spaces: Embodiment and being in virtual environments. International Journal of Performance Arts and Digital Media, 3(2-3), pp.123-138.

Muluneh, W., 2003. Impact of population pressure on land use and land cover changes, agricultural system and income diversification in west Gurague land, Ethiopia. Norwegian University of Science and Technology (NTNU): Trondheim

Musa, M.K. and Odera, P.A., 2015. Land use land cover changes and their effects on agricultural land a case study of Kiambu County Kenya.

Mushir A, Kedru S (2012) Soil and water conservation management through indigenous and traditional practices in Ethiopia: a case study. Ethiopian Journal of Environmental Studies and Management 5: 343-355. Link: https://tinyurl.com/y65e6llw.

Navas, A., Valero-Garcés, B., Gaspar, L. and Machín, J., 2009. Reconstructing the history of sediment accumulation in the Yesa reservoir: an approach for management of mountain reservoirs. Lake and Reservoir Management, 25(1), pp.15-27.

Negessa Gadisa and Tesfaye Midega  2021. Soil and water conservation measures in Ethiopia: Importance and adoption challenges. World Journal of Agricultural & Soil Science, 6(3).

Neitsch, S.L., Arnold, J.G. and Srinivasan, R., 2002. Pesticides fate and transport predicted by the soil and water assessment tool (SWAT). Atrazine, Metolachlor and Trifluralin in the Sugar Creek Watershed: BRC Report, 3.

Nigussie Haregeweyn, , Tsunekawa, A, Poesen, J, Mitsuru Tsubo, Derege Tsegaye Meshesha, Ayele Almaw Fenta, Nyssen, J, and Enyew Adgo 2017. Comprehensive assessment of soil erosion risk for better land use planning in river basins: Case study of the Upper Blue Nile River. Science of the Total Environment, 574:  pp.95-108 .

Nigussie Haregeweyn, Bekure Melesse Tsunekawa, A., Tsubo, M. Derege Meshesha Balana, B.B., 2012. Reservoir sedimentation and its mitigating strategies: a case study of Angereb reservoir (NW Ethiopia). Journal of Soils and Sediments, 12(2), pp.291-305.

Nigussie Haregeweyn, Jean Poesen, Jan Nyssen, J. De Wit, Mitiku Haile.,Govers, G. and Deckers, S., 2006. Reservoirs in Tigray (Northern Ethiopia): characteristics and sediment deposition problems. Land degradation & development, 17(2), pp.211-230.

Nigussie Haregeweyn, Tsunekawa, A., Nyssen, J., Poesen, J., Tsubo, M.,Derege Tsegaye Meshesha Schütt, B Enyew Adgo, and Firew Tegegne 2015. Soil erosion and conservation in Ethiopia: a review. Progress in Physical Geography, 39(6), pp.750-774.

Nyssen J, Poesen J, Moeyersons J, Haile M, Deckers J (2007) Dynamics of soil erosion rates and controlling factors in the northern Ethiopian highlands towards a sediment budget Earth Surf Process Land f. https://doi.org/10.1002/esp.1569

Oldeman L, Van Lynden J and Van Engelen (1995). An international methodology for soil degradation assessment and for a soils and terrain digital database (SOTER). Paper presented at the regional workshop on assessment and monitoring of land degradation, Cairo, May 13-15, 1995.

Ostovari, Y., Ghorbani-dashtaki, S., Bahrami, H., Naderi, M., Alexandre, J. and Dematte, M. 2017. Soil loss estimation using RUSLE model, GIS and remote sensing techniques : A case study from the Dembecha watershed, Northwestern Ethiopia. Geoderma Reg. 11: 28-36.

Panagos P, Borrelli P, Meusburger K, van der Zanden EH, Poesen J, Alewell C (2015) Modelling the efect of support practices (P-factor) on the reduction of soil erosion by water at European scale. Environ Sci Policy 51:pp 23–34.

Pathak, P., Chourasia, A.K., Wani, S.P. and Sudi, R., 2013. Multiple impact of integrated watershed management in low rainfall semi-arid region: A case study from eastern Rajasthan, India. Journal of Water Resource and Protection, 5(1), pp.27-36.

Phinzi, K. and Ngetar, N.S., 2019. The assessment of water-borne erosion at catchment level using GIS-based RUSLE and remote sensing: A review. International Soil and Water Conservation Research, 7(1), pp.27-46.

Piccarreta M, Capolongo D, Boenzi F, Bentivenga M (2006) Implications of decadal changes in precipitation and land use policy to soil erosion in Basilicata, Italy. CATENA 65(2):138–151.

Prasannakumar, V., Vijith, H., Abinod, S. and Geetha, N. 2012. Estimation of soil erosion risk within a small mountainous sub-watershed in Kerala, India, using Revised Universal Soil Loss Equation (RUSLE) and geo-information technology. Geosci Front. 3(2):209-215.

Quan, X., He, J., Cai, Q., Sun, L., Li, X. & Wang, S. 2020. Soil erosion and deposition characteristics of slope surfaces for two loess soils using indoor simulated rainfall experiment. Soil & Tillage Research 204: 104714.

Rahdary, V., Najfabdai, S.M. and Khajeddin, S.J., 2008. Land Use and Land Cover Change Detection of Mouteh Wildlife Refuge Using Remotely Sensed Data and Geographic Information System 1.

Rasool R, Fayaz A, ul Shafiq M, Singh H, Ahmed P (2021) Land use land cover change in Kashmir Himalaya: linking remote sensing with an indicator based DPSIR approach. Ecol Indic 125:107447

Rawat, J.S. and Kumar, M., 2015. Monitoring land use/cover change using remote sensing and GIS techniques: A case study of Hawalbagh block, district Almora, Uttarakhand, India. The Egyptian Journal of Remote Sensing and Space Science, 18(1), pp.77-84.

Renard, K.G., Foster, G.R., Weesies, G.A., McCool, D.K. and Yoder, D.C., 1996. Predicting soil erosion by water: A guide to conservation planning with the Revised Universal Soil Loss Equation (RUSLE). Agriculture handbook, 703.

Reusing M, Schneider T, Ammer U (2000) Modeling soil loss rates in the Ethiopian highlands by integration of high resolution MOMS02/D2-stereo-data in a GIS. Int J Remote Sens 21(9): pp 1885–1896.

Rientjes, T.H.M., Haile, A.T., Kebede, E., Mannaerts, C.M.M., Habib, E. and Steenhuis, T.S., 2011. Changes in land cover, rainfall and stream flow in Upper Gilgel Abbay catchment, Blue Nile basin–Ethiopia. Hydrology and Earth System Sciences, 15(6): pp.1979-1989.

Ristić, R., Kostadinov, S., Radić, B., Trivan, G. and Nikić, Z., 2012.Torrential floods in Serbia–man made and natural hazards.In 12th congress interpraevent.

Rodrigo Comino, J., Brings, C., Lassu, T., Iserloh, T., Senciales, J.M., Martínez Murillo, J.F., Ruiz Sinoga, J.D., Seeger, M. and Ries, J.B., 2015. Rainfall and human activity impacts on soil losses and rill erosion in vineyards (Ruwer Valley, Germany). Solid Earth, 6(3), pp.823-837.

Römkens MJM, Young RA, Poesen JWA, McCool DK, El-Swaify SA, Bradford JM (1997) Soil erodibility factor (K). Predicting Soil Erosion by Water: A Guide to Conservation, edited by: Renard, KG, Foster, GR, Weesies, GA, McCool, DK, and Yoder, DC, coordinators, Planning with the Revised Universal Soil Loss Equation (RUSLE), US Department of Agriculture, Agriculture Handbook, (703): pp. 65–99

Sahle M, Saito O, Fürst C, Yeshitela K (2019) Quantifying and mapping of water-related ecosystem services for enhancing the security of the food-water-energy nexus in tropical data–sparse catchment. Sci Total Environ 646:573–586.

Samuale Tesfaye, Etefa Guyassa, Joseph Raj, A, Emiru Birhane, and Gebeyehu Taye Wondim., 2014. Land use and land cover change, and woody vegetation diversity in human driven landscape of Gilgel Tekeze Catchment, Northern Ethiopia. International Journal of Forestry Research, 2014.

Schwab, M.E., Kapfhammer, J.P. and Bandtlow, C.E., 1993. Inhibitors of neurite growth. Annual review of neuroscience, 16(1), pp.565-595.

Seleshi Bekele., McCartney, M., Steenhuis, T., Ahmed, A. (2008) A study at hydrology, sediment, and the Blue Nile basin's water supplies International Water Conservation Institute, Colombo, Sri Lanka Working Document of the IWMI 131.P.p87.

Setegn, S.G., Dargahi, B., Srinivasan, R. and Melesse, A.M., 2010. Modeling of Sediment Yield From Anjeni‐Gauged Watershed, Ethiopia Using SWAT Model 1. JAWRA Journal of the American Water Resources Association, 46(3), pp.514-526.

Setegn, Shimelis G., Ragahavan Srinivasan, Assefa M. Melesse, and Bijan Dargahi. 2010. SWAT model application and prediction uncertainty analysis in the Lake Tana Basin, Ethiopia. Hydrological Processes: An International Journal, 24(3), pp.357-367

Shahin, M.M.A., 1993. An overview of reservoir sedimentation in some African river basins. IAHS publication, pp.93-93.

Sharma, A., Tiwari, K.N. and Bhadoria, P.B.S., 2011. Effect of land use land cover change on soil erosion potential in an agricultural watershed. Environmental monitoring and assessment, 173(1), pp.789-801.

Sharp R, Tallis HT, Ricketts T, Guerry AD, Wood SA, Chaplin-Kramer R, Nelson E, Ennaanay D, Wolny S, Olwero N, Vigerstol K, Pennington D, Mendoza G, Aukema J, Foster J, Forrest J, Cameron D, Arkema K, Lonsdorf E, Kennedy C, Verutes G, Kim CK, Guannel G, Papenfus M, Toft J, Marsik M, Bernhardt J, Grifn R, Glowinski K, Chaumont N, Perelman A, Lacayo M, Mandle L, Hamel P, Vogl AL, Rogers L, Bierbower W, Denu D, Douglass J (2018). InVEST 3.50.post225+h6aabb7740548 User’s Guide. The Natural Capital Project, Stanford University, University of Minnesota, The Nature Conservancy, and World Wildlife Fund.

Shimelis G. Setegn, Srinivasan, R. and Dargahi, B., 2008. Hydrological modelling in the Lake Tana Basin, Ethiopia using SWAT model. The Open Hydrology Journal, 2(1).

Shrestha, S., 2012. Drivers of Land Use Change: A Study from Sundarijal Catchment, Shivapuri Nagarjun National Park (Doctoral dissertation, MSc. thesis, Kathmandu University).

Sisay Nune Hailemariam,, Teshome Soromessa, and Demel Teketay, 2016. "Land use and land cover change in the bale mountain eco-region of Ethiopia during 1985 to 2015." Land 5, no. 4 pe 41.

Soil Conservation Research Programme (SCRP). (2000). Area of Maybar, Wello, Ethiopia: Long-term Monitoring of the Agricultural Environment 1981 - 1994, Soil Erosion and Conservation Database, Centre for Development and Environment University of Berne Switzerland, The Ministry of Agriculture Ethiopia,, Addis Ababa, Ethiopia.

Solomon Gebreyohannis., Ayele Taye., Bishop, K. (2010). Forest cover and stream flow in a headwater of the Blue Nile: Complementing observational data analysis with community perception. Ambio. 39 (4):284-294.

Sorooshian, S., Duan, Q. and Gupta, V.K., 1993. Calibration of rainfall‐runoff models: Application of global optimization to the Sacramento Soil Moisture Accounting Model. Water resources research, 29(4), pp.1185-1194.

Stocking, M, 2003. Tropical soils and food security: the next 50 years. Science, 302: 1356-1359.

Swarnkar S, Malini A, Tripathi S. and Sinha R., 2018. Assessment of uncertainties in soil erosion and sediment yield estimates at ungauged basins: an application to the Garra River basin, India. Hydrology and Earth System Sciences, 22(4), pp.2471-2485.

Syed Ahmad Ali and Hamelmal Hagos .,2016. Estimation of soil erosion using USLE and GIS in Awassa Catchment, Rift valley, Central Ethiopia. Geoderma Regional, 7(2), pp.159-166.

Syvitski, J.P. and Milliman, J.D., 2007. Geology, geography, and humans battle for dominance over the delivery of fluvial sediment to the coastal ocean. The Journal of Geology, 115(1), pp.1-19.

Tadesse Amsalu and Abebe Mengaw. (2014). GIS based soil loss estimation using RUSLE Model: The case of Jabi Tehinan Woreda, ANRS. Ethiopia. Natural Resources. 5:616-626.

Tamene, L., Park, S.J., Dikau, R. and Vlek, P.L.G., 2006 analysis of the factors that influence sediment yield variability in northern Ethiopia's highlands Geomorphology

Tarboton DG (1997) A new methods for the determination of flow directions and upslope areas in grid digital elevation models. Water Resour Res 33(2): pp 309–319.

Tegegne Molla and Biniam Sisheber., 2017. Estimating soil erosion risk and evaluating erosion control measures for soil conservation planning at Koga watershed in the highlands of Ethiopia. Solid Earth, 8(1), pp.13-25.

Tekalegn Ayele, Woldesenbet Nadir Ahmed Elagib, Lars Ribbe, and Jürgen Heinrich2017. Hydrological responses to land use/cover changes in the source region of the Upper Blue Nile Basin, Ethiopia. Science of the Total Environment, 575, pp.724-741

Teklu Erkossa, Alemayehu Wudneh, Bamlaku Desalegn, and Getnet Taye., 2015. Linking soil erosion to on-site financial cost: lessons from watersheds in the Blue Nile basin. Solid Earth, 6(2), pp.765-774. .

Temesgen Gashaw., Taffa Tulu., Mekuria Argaw. (2017). Erosion risk assessment for prioritization of conservation measures in Geleda watershed, Blue Nile basin, Ethiopia. Environ Syst Res. 6(1):1-14.

Temesgen, Gashaw, Taffa Tulu, Mekuria Argaw, and Abeyou W. Worqlul.,2019. Modeling the impacts of land use–land cover changes on soil erosion and sediment yield in the Andassa watershed, upper Blue Nile basin, Ethiopia. Environmental Earth Sciences, 78(24), pp.1-22

Temesgen, Gashaw, Taffa Tulu, Mekuria Argaw, and Abeyou W. Worqlul., 2018. Modeling the hydrological impacts of land use/land cover changes in the Andassa watershed, Blue Nile Basin, Ethiopia. Science of the Total Environment, 619: pp.1394-1408.

Terefe,Tolessa, Chala Dechassa, Belay Simane, Bamlaku Alamerew, and Moges Kidane 2020. Land use/land cover dynamics in response to various driving forces in Didessa sub-basin, Ethiopia. GeoJournal, 85(3): pp.747-760.

Theobald, M.R., Løfstrøm, P., Walker, J., Andersen, H.V., Pedersen, P., Vallejo, A. and Sutton, M.A., 2012. An intercomparison of models used to simulate the short-range atmospheric dispersion of agricultural ammonia emissions. Environmental Modelling & Software, 37, pp.90-102.

Thomlinson, J.R., Bolstad, P.V. and Cohen, W.B., 1999. Coordinating methodologies for scaling landcover classifications from site-specific to global: Steps toward validating global map products. Remote Sensing of Environment, 70(1), pp.16-28.

Tsegaye Diress, D., Moe, S.R., Vedeld, P. and Ermias Aynekulu 2010. Land-use/cover dynamics in Northern Afar rangelands, Ethiopia. Agriculture, ecosystems & environment, 139(1-2), pp.174-180.

Tully K., Sullivan C., Weil.R,  and Sanchez P., 2015. The state of soil degradation in Sub-Saharan Africa: Baselines, trajectories, and solutions. Sustainability, 7(6), pp 6523-6552.

Turner, B. L., Skole, D., Sanderson, S., Fischer, G., Fresco, L. and Leemans, R. 1995. LandUse and Land-cover change Science/Research Plan. Joint publication of the International Geosphere-Biosphere Programme (Report No. 35) and the Human Dimensions of Global Environmental Change Programme (Report No. 7). Stockholm: Royal Swedish Academy of Science.

UNEP (2013) Adaptation to Climate-change Induced Water Stress in the Nile Basin: A Vulnerability Assessment Report. Nairobi, Kenya: Division of Early Warning and Assessment (DEWA), United Nations Environment Programme (UNEP). Link: https://tinyurl.com/y2memkaf

Van Remortel, R.D., Hamilton, M.E. and Hickey, R.J., 2001. Estimating the LS factor for RUSLE through iterative slope length processing of digital elevation data within Arclnfo grid. Cartography, 30(1): pp.27-35.

Varamesh, S., Hosseini, S.M. and Rahimzadegan, M., 2017. Detection of land use changes in northeastern Iran by landsat satellite data. Applied Ecology and Environmental Research, 15(3), pp.1443-1454.

Verheye, W., 2009. Land cover, land use and the global change. Encyclopedia of land use, land cover and soil sciences-land cover, land use and the global change, pp.45-80.

Verheye, W., 2010. Land use management. Land Use, Land Cover and Soil Science IV. University Gent, Belgium.

Vigiak O, Borselli L, Newham LTH, McInnes J, Roberts AM (2012) Comparison of conceptual landscape metrics to defne hillslope-scale sediment delivery ratio. Geomorphology 138(1): pp 74–88.

Wagesho, N., 2014. Catchment dynamics and its impact on runoff generation: coupling watershed modelling and statistical analysis to detect catchment responses. International Journal of Water Resources and Environmental Engineering, 6 (2), 73–87. doi:10. 5897/IJWREE2013.0449.

Wagner, P.D., Kumar, S., and Schneider, K., 2013. An assessment of land use change impacts on the water resources of the Mula and Mutha Rivers catchment upstream of Pune, India. Hydrology and Earth System Science, 17, 2233–2246. doi:10.5194/hess-17- 2233-2013.

Wakjira Takala Dibaba, Tamene Adugna Demissie, Miegel, K., 2020. Drivers and implications of land use/land cover dynamics in Finchaa catchment, northwestern Ethiopia. Land, 9(4), p.113.

Wang C, Wang Y, Wang R, Zheng P (2018) Modeling and evaluating land-use/land-cover change for urban planning and sustainability: a case study of Dongying city, China. J Clean Prod 172:1529–1534

WIREs Climate Change, 2014, Interactions between Climate Change and Land use Change on Biodiversity: Attribution Problems, Risks, and Opportunities (5): 317–335.

Wischmeier, W.H. and Smith, D.D., 1978. Predicting rainfall erosion losses a guide to conservation planning, Department of Agriculture Science and Education Administration No. 537

Woldeamlak Bewket, 2003. Towards integrated watershed management in highland Ethiopia: the Chemoga watershed case study. Wageningen University and Research.

Woldeamlak Bewket. 2002. Land covers dynamics since the 1950s in Chemoga      watershed, Blue Nile basin, Ethiopia. Mountain research and development, 22(3): pp.263-269.

Woldeamlak, Bewket and Ermias Teferi .,2009. Assessment of soil erosion hazard and prioritization for treatment at the watershed level: case study in the Chemoga watershed, Blue Nile basin, Ethiopia. Land degradation & development, 20(6), pp.609-622.

Woldeamlak, Bewket, and Solomon Abebe., 2013. Land-use and land-cover change and its environmental implications in a tropical highland watershed, Ethiopia. International journal of environmental studies, 70(1), pp.126-139.

Woldeamlak, Bewket, and Sterk, G., 2002. Farmers' participation in soil and water conservation activities in the Chemoga watershed, Blue Nile basin, Ethiopia. Land Degradation & Development, 13(3), pp.189-20.

Xu, C., Dai, F., Xu, X. and Lee, Y.H., 2012. GIS-based support vector machine modeling of earthquake-triggered landslide susceptibility in the Jianjiang River watershed, China. Geomorphology, 145, pp.70-80.

Yang, D.; Kanae, S.; Oki, T.; Koike, T.; Musiake, K, 2003. Global potential soil erosion with reference to land use and climate changes. Hydrol. Process., 17, 2913–2928, doi:10.1002/hyp.1441.

Yihenew G. Selassie., Yihenew Belay. (2013). Costs of nutrient losses in priceless soils eroded from the highlands of Northwestern Ethiopia. J Agric Sci. 5 (7):1916-9752.

Yihun, B.B., 2016. Setit-Humera: A Blister on Ethio–Sudanese Boundary Disputation. Journal of Borderlands Studies, 31(1), pp.107-122.

Yue-Qing, X., Xiao-Mei, S., Xiang-Bin, K., Jian, P. and Yun-Long, C., 2008. Adapting the RUSLE and GIS to model soil erosion risk in a mountains karst watershed, Guizhou Province, China. Environmental monitoring and assessment, 141, pp.275-286.

BIOGRAPHICAL SKETCH

He was born in north wollo Administrative Zone, Gubalafto District particularly

Woyineye Kebele in October19, 1991 G.C. In 1997, he joined woyineye junior elementary school where he completed attending education until sixth grade in June 2002, from grade 7-8 Bles junior elementary school in June 2004 G.C and completed his secondary and preparatory school education at woldia in June 2008 G.C. Then, he joined Wollo University in October 2009 and received his BSc degree in the field of "Natural Resource Management" in June 2011 G.C. After his BSc Degree, he has employed at Agricultural and Rural Development Office of Gubalafto District the position of "Natural Resource Management expert" and he was served from November 27, 2012, up to 2020 G.C, for eight years. In 2020 he joined a master's degree program in "Environment and Climate Change" at Bahir Dar University, College of Agriculture and Environmental Sciences and he did his thesis work entitled " effects of land use land cover change on soil erosion and sediment yield: A Case Study of maybarWatershed, NorthEastern, Ethiopia" under the supervision of Lewoye Tsegaye (Associate. prof).